LIN Protocol: Frame Data & Protocol Analysis

1. Bit-Level Anatomy of a LIN Frame

A complete LIN communication transaction is termed a Frame. Every LIN frame is strictly divided into two logical sections:

Frame Header: Generated and transmitted exclusively by the Master Node.

Frame Response: Generated and transmitted by a designated Slave Node (or by the Master itself when broadcasting commands).

¦<------------------- Master Header ------------------>¦<----------------- Slave Response ----------------->¦ ┌──────────────────────┬─────────────┬─────────────────┬────────────────┬────────────────┬─────────────────┐ │ Synch Break Field │ Synch Byte │ Protected ID │ Data Byte 1 │ Data Byte N │ Checksum Byte │ ├──────────┬───────────┼─────────────┼─────────────────┼────────────────┼────────────────┼─────────────────┤ │ Dominant │ Delimiter │ 0x55 │ 6-bit ID + 2P │ 1 Start, 8 Data│ 1 Start, 8 Data│ 1 Start, 8 Data │ │ >= 13 b │ >= 1 bit │ 8-N-1 UART │ 8-N-1 UART │ 1 Stop bit │ 1 Stop bit │ 1 Stop bit │ └──────────┴───────────┴─────────────┴─────────────────┴────────────────┴────────────────┴─────────────────┘ ¦ ¦<------- Inter-byte Space ---->¦<------- Inter-byte Space ------>¦

1.1 The Master Header Components

A. Synch Break Field (Break)

Dominant Period: The Master holds the bus Dominant for a minimum of 13 bit periods (typically 13 \dots 26 bit times). Because standard UART bytes cannot hold a zero longer than 9 or 10 bit times (Start + 8 data zeros before the mandatory Stop bit), a ≥ 13-bit dominant pulse is a deliberate framing violation that alerts every slave that a new frame has commenced.

Break Delimiter: Immediately follows the dominant break with at least 1 bit period of Recessive ('1').

B. Synch Byte Field

Transmitted as a standard 8-N-1 UART character with value `0x55` (01010101b).

In UART transmission (LSB first with a dominant Start bit and recessive Stop bit), 0x55 generates 5 identical falling edges:

UART Line: Idle ──┐ 0 ┌── 1 ┌── 0 ┌── 1 ┌── 0 ┌── 1 ┌── 0 ┌── 1 ┌── Stop (1) │ │ │ │ │ │ │ │ │ │ Falling Edges: └──1──┘ └──2──┘ └──3──┘ └──4──┘ └──5──┘ ¦<----------------- Exactly 8 Bit Periods ---------------->¦

Dynamic Auto-Baud: Slaves measure the precise elapsed time between Edge 1 (falling edge of the Start bit) and Edge 5 (falling edge of bit 7). Dividing this interval by 8 yields the Master's exact bit period (T_{bit), allowing slaves with cheap RC oscillators to lock their baud rates!

C. Protected Identifier (PID) Field

The PID is an 8-bit byte containing a 6-bit Frame Identifier (`ID0` to `ID5`) and 2 Parity Bits (`P0`, `P1`):

Bit: 7 6 5 4 3 2 1 0 ┌─────┬─────┬─────┬─────┬─────┬─────┬─────┬─────┐ │ P1 │ P0 │ ID5 │ ID4 │ ID3 │ ID2 │ ID1 │ ID0 │ └─────┴─────┴─────┴─────┴─────┴─────┴─────┴─────┘ ¦<-- Parity ->¦<----------- 6-bit ID ------------>¦

Parity Calculation Equations:

Parity Bit 0 ($P0$) is an even parity over bits 0, 1, 2, and 4:

P0 = ID0 \oplus ID1 \oplus ID2 \oplus ID4

Parity Bit 1 ($P1$) is an inverted odd parity over bits 1, 3, 4, and 5:

P1 = \overline{ID1 \oplus ID3 \oplus ID4 \oplus ID5} = 1 \oplus (ID1 \oplus ID3 \oplus ID4 \oplus ID5)

/* C Implementation of LIN PID Parity Generation */ uint8_t lin_calculate_pid(uint8_t raw_id) { uint8_t id = raw_id & 0x3F; uint8_t id0 = (id >> 0) & 1; uint8_t id1 = (id >> 1) & 1; uint8_t id2 = (id >> 2) & 1; uint8_t id3 = (id >> 3) & 1; uint8_t id4 = (id >> 4) & 1; uint8_t id5 = (id >> 5) & 1; uint8_t p0 = id0 ^ id1 ^ id2 ^ id4; uint8_t p1 = !(id1 ^ id3 ^ id4 ^ id5); return (uint8_t)((p1 << 7) ¦ (p0 << 6) ¦ id); }

[NOTE] The parity bits guarantee that all 1-bit bit errors in the identifier are detected and 2-bit bit errors cannot result in a false PID match.

1.2 LIN 64-Identifier Allocation Map

The 6-bit identifier yields 64 total values (0x00 \dots 0x3F), strictly cataloged by the LIN specification:

| Raw ID Range | PID Values | Frame Type / Designation | Usage & Characteristics |
| --- | --- | --- | --- |
| `0x00` – `0x3B` (0 - 59) | 0x00 – 0xBB | Unconditional Frames | Carries normal real-time sensor/actuator payload (1 \dots 8 bytes). |
| `0x3C` (60) | `0x3C` (P0=0, P1=0) | Master Request Frame | Carries diagnostic commands, node configuration, and sleep requests from Master. Always uses Classic Checksum. |
| `0x3D` (61) | `0x7D` (P0=1, P1=0) | Slave Response Frame | Designated slave transmits diagnostic telemetry or EOL configuration responses. Always uses Classic Checksum. |
| `0x3E` (62) | `0xFE` (P0=1, P1=1) | User-Defined Extension | OEM-specific proprietary test harness frame. |
| `0x3F` (63) | `0xBF` (P0=0, P1=1) | Reserved | Reserved for future protocol enhancements. |

2. Slave Response & Checksum Mathematics

The response consists of 1 to 8 Data Bytes followed by an 8-bit Checksum. Data is transmitted LSB first in standard UART bytes (1 Start bit, 8 Data bits, 1 Stop bit).

2.1 Classic vs. Enhanced Checksum Algorithms

| Checksum Type | Specification | Included Fields | Used By |
| --- | --- | --- | --- |
| Classic Checksum | LIN 1.3 | Inverted 8-bit sum of Data Bytes ONLY | Legacy LIN 1.3 slaves, and Diagnostic Frames (`0x3C`, `0x3D`) in all versions. |
| Enhanced Checksum | LIN 2.x & ISO 17987 | Inverted 8-bit sum of PID + Data Bytes | Modern LIN 2.x / ISO 17987 unconditional and sporadic frames. |

2.2 Mathematical Algorithm (One's Complement Carry Addition)

The checksum algorithm performs an 8-bit modulo-256 addition where every overflow carry bit is immediately added back into the least significant bit (known as an end-around carry or one's complement sum):

\text{Sum} = \text{PID (if Enhanced)} + \sum_{i=1}^{N} \text{Data}_i + \text{Carries}

\text{Checksum} = \neg (\text{Sum} \ \& \ \text{0xFF}) = (\sim\text{Sum}) \ \& \ \text{0xFF}

Verification at Receiver:

Adding the received checksum to the calculated sum must equal `0xFF`:

\text{Sum} + \text{Received\_Checksum} = 0\text{xFF}

/* Production-Grade LIN Checksum Implementation in C */ uint8_t lin_calculate_checksum(uint8_t pid, const uint8_t *data, uint8_t length, bool is_classic) { uint16_t sum = 0; /* Enhanced checksum includes PID; Classic does not */ if (!is_classic) { sum += pid; } for (uint8_t i = 0; i < length; i++) { sum += data[i]; /* If 8-bit overflow occurred, wrap carry into LSB */ if (sum > 0xFF) { sum = (sum & 0xFF) + 1; } } /* Bitwise NOT (one's complement inversion) */ return (uint8_t)(~sum & 0xFF); }

3. LIN Communication Frame Types

Unconditional Frames:

The standard communication vehicle. The Master sends a header with PID 0x00 \dots 0x3B. The designated slave (or master) immediately responds in the same time slot.

Event-Triggered Frames:

Allows multiple slave nodes to share a single scheduled time slot to report infrequent events (e.g., any door lock status changing).

If only one slave responds, the Master receives the frame normally.

If two slaves respond simultaneously, their data collides on the wire, resulting in a Checksum Error.

The Master detects the checksum mismatch and automatically invokes a Collision-Resolving Schedule Table, polling each participating slave individually via unconditional frames.

Sporadic Frames:

Slots reserved by the Master for frames that are only transmitted when new data is available. If no update exists, the slot remains empty/idle.

Diagnostic Frames (`0x3C` & `0x3D`):

Carries ISO 14229 UDS or node configuration payloads using the LIN Transport Layer (LIN TP).

4. LIN Transport Layer (LIN TP / ISO 17987-2)

When diagnostic or calibration data exceeds 8 bytes, the LIN Transport Layer segments the payload across multiple consecutive frames using a Protocol Control Information (PCI) byte:

┌──────────┬──────────┬──────────┬──────────┬────────────────────────────┐ │ NAD │ PCI │ SID / │ Param 1 │ Param N │ │ (1 Byte) │ (1 Byte) │ D1 Byte │ (1 Byte) │ (4 Bytes) │ └──────────┴──────────┴──────────┴──────────┴────────────────────────────┘

Single Frame (SF): PCI byte 0x01 to 0x06 indicates payload fits in 1 to 6 bytes.

First Frame (FF): PCI byte 0x1_ indicates the start of a multi-frame transfer, encoding total byte length.

Consecutive Frame (CF): PCI byte 0x2_ carries subsequent data segments with a rolling sequence counter.

5. Oscilloscope / Logic Analyzer Waveform Walkthrough

The following trace illustrates a captured LIN frame transmitting PID 0xA3 (raw ID 0x23), payload [0x01, 0x02], and Enhanced Checksum 0x59:

Line State: ¦<- Break (>=13b) ->¦ Delim ¦<--- Sync (0x55) --->¦ ¦<--- PID (0xA3) ---->¦ ¦<-- Data 0x01 -->¦ ¦<-- Chk 0x59 -->¦ 12V (VBAT):──────┐ ┌───────┐ ┌─┐ ┌─┐ ┌─┐ ┌─┐ ┌───┐ ┌───┐ ┌─┐ ┌─────────┐ ┌───┐ ┌──┐ ┌─┐ ┌───┐ ┌─── │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ GND (0V): └─────────────┘ └─┴─┴─┴─┴─┴─┴─┴─┴─┘ └───┘ └───┴─┴─┘ └───┘ └─────────────┴──┴─┴─┴─┘ └─┴─── Bits: 0000000000000 1 0 1 0 1 0 1 0 1 0 1 0 11000101 (0xA3) 1 0 10000000 1 0 10011010 1 Fields: Dominant Break Delim Start, 0x55, Stop St. P1 P0 ID5..ID0 Stop St. 0x01 Stop St. 0x59 Stop Decoded: [ BREAK FIELD ] [ SYNCHRONIZATION ] [ PROTECTED IDENTIFIER] [ PAYLOAD BYTE 1 ] [ ENHANCED CHKSUM ]

6. Microcontroller Firmware Implementation (UART Driver)

Standard microcontrollers (STM32, NXP S32K, Microchip AVR/PIC) utilize hardware UART/USART peripherals configured for LIN mode:

/* stm32_lin_master.c - Transmitting a LIN Frame Header using Hardware UART */ #include "stm32f4xx_hal.h" extern UART_HandleTypeDef huart1; void LIN_Master_SendHeader(uint8_t lin_id) { uint8_t sync_byte = 0x55; uint8_t pid = lin_calculate_pid(lin_id); /* 1. Request Hardware LIN Break Transmission (generates 13-bit dominant break) */ HAL_LIN_SendBreak(&huart1); /* 2. Transmit Sync Byte (0x55) */ HAL_UART_Transmit(&huart1, &sync_byte, 1, 10); /* 3. Transmit Protected Identifier (PID) */ HAL_UART_Transmit(&huart1, &pid, 1, 10); /* Master header complete. Designated slave now responds on the bus. */ }