NFC Embedded Firmware Architecture, Linux Driver Integration & Debugging Guide

1. Embedded Firmware Driver Architecture

To achieve clean portability across microcontroller platforms (STM32, ESP32, NXP LPC/i.MX RT, Nordic nRF), the NFC software stack is structured into four decoupled layers:

+-----------------------------------------------------------------------------------+ ¦ APPLICATION LAYER ¦ ¦ (NDEF URI Launcher, Access Control Gate, Transit HCE Payment) ¦ +-----------------------------------------------------------------------------------+ ¦ +-----------------------------------------------------------------------------------+ ¦ PROTOCOL ENGINE LAYER ¦ ¦ (ISO/IEC 14443-3 Poller & Anti-collision, ISO 14443-4 T=CL, ISO 7816-4 APDU) ¦ +-----------------------------------------------------------------------------------+ ¦ +-----------------------------------------------------------------------------------+ ¦ DEVICE DRIVER / AFE LAYER ¦ ¦ (Register Maps, FIFO Push/Pull, BitFraming Control, CRC16_A Engine, RF Power) ¦ +-----------------------------------------------------------------------------------+ ¦ +-----------------------------------------------------------------------------------+ ¦ HARDWARE ABSTRACTION LAYER (HAL) ¦ ¦ (SPI_Transfer, GPIO_CS_Assert, GPIO_CS_Deassert, IRQ_ISR, Timer_Delay_ms) ¦ +-----------------------------------------------------------------------------------+

2. Production-Grade Embedded C Driver Implementation

Below is a complete, modular, production-ready implementation in C demonstrating SPI HAL abstraction, register access, FIFO burst transfers, RF field control, 7-bit REQA framing, and the complete ISO 14443-3 anti-collision loop.

2.1 Hardware Abstraction Layer Header (nfc_hal.h)

#ifndef NFC_HAL_H #define NFC_HAL_H #include <stdint.h> #include <stdbool.h> #include <stddef.h> /** * @brief Initialize SPI peripheral, CS pin, Reset pin, and IRQ line. */ void nfc_hal_init(void); /** * @brief Assert Chip Select (drive CS# LOW). */ void nfc_hal_cs_low(void); /** * @brief Deassert Chip Select (drive CS# HIGH). */ void nfc_hal_cs_high(void); /** * @brief Full-duplex synchronous SPI byte transfer. * @param tx_byte Byte to transmit onto MOSI. * @return Byte clocked in from MISO. */ uint8_t nfc_hal_spi_transfer_byte(uint8_t tx_byte); /** * @brief Synchronous multi-byte SPI transfer (burst mode). * @param tx_buf Buffer to send (can be NULL if only reading). * @param rx_buf Buffer to receive (can be NULL if only writing). * @param length Total number of bytes to transfer. */ void nfc_hal_spi_transfer(const uint8_t *tx_buf, uint8_t *rx_buf, size_t length); /** * @brief Blocking millisecond delay. */ void nfc_hal_delay_ms(uint32_t ms); /** * @brief Blocking microsecond delay. */ void nfc_hal_delay_us(uint32_t us); /** * @brief Drive hardware reset pin. * @param state true = assert reset (LOW), false = release reset (HIGH). */ void nfc_hal_reset_pin(bool state); #endif /* NFC_HAL_H */

2.2 Transceiver Driver Implementation (nfc_transceiver.c)

#include "nfc_hal.h" #include <string.h> /* Standard MFRC522 / Register-Driven Transceiver Register Addresses */ #define REG_COMMAND 0x01 #define REG_COMIEN 0x02 #define REG_COMIRQ 0x04 #define REG_ERROR 0x06 #define REG_STATUS2 0x08 #define REG_FIFO_DATA 0x09 #define REG_FIFO_LEVEL 0x0A #define REG_WATER_LEVEL 0x0B #define REG_CONTROL 0x0C #define REG_BIT_FRAMING 0x0D #define REG_COLL 0x0E #define REG_MODE 0x11 #define REG_TX_CONTROL 0x14 #define REG_TX_ASK 0x15 #define REG_CRC_RESULT_HIGH 0x21 #define REG_CRC_RESULT_LOW 0x22 #define REG_T_MODE 0x2A #define REG_T_PRESCALER 0x2B #define REG_T_RELOAD_HIGH 0x2C #define REG_T_RELOAD_LOW 0x2D /* Command Codes */ #define CMD_IDLE 0x00 #define CMD_MEM 0x01 #define CMD_GEN_RANDOM_ID 0x02 #define CMD_CALC_CRC 0x03 #define CMD_TRANSMIT 0x04 #define CMD_NO_CMD_CHANGE 0x07 #define CMD_RECEIVE 0x08 #define CMD_TRANSCEIVE 0x0C #define CMD_MF_AUTHENT 0x0E #define CMD_SOFT_RESET 0x0F /* Status Codes */ typedef enum { NFC_STATUS_OK = 0, NFC_STATUS_TIMEOUT, NFC_STATUS_COLLISION, NFC_STATUS_CRC_ERROR, NFC_STATUS_FIFO_OVERFLOW, NFC_STATUS_PROTOCOL_ERROR } nfc_status_t; /* Tag UID Structure */ typedef struct { uint8_t uid_bytes[10]; uint8_t uid_length; /* 4 (Single), 7 (Double), or 10 (Triple) */ uint8_t sak; /* Select Acknowledge byte */ } nfc_tag_t; /* ========================================================================== */ /* Register Read / Write Helpers over SPI */ /* ========================================================================== */ void nfc_write_reg(uint8_t reg_addr, uint8_t value) { nfc_hal_cs_low(); /* Format: Bit 7 = 0 (Write), Bits 6..1 = Address, Bit 0 = 0 */ nfc_hal_spi_transfer_byte((reg_addr << 1) & 0x7E); nfc_hal_spi_transfer_byte(value); nfc_hal_cs_high(); } uint8_t nfc_read_reg(uint8_t reg_addr) { uint8_t value; nfc_hal_cs_low(); /* Format: Bit 7 = 1 (Read), Bits 6..1 = Address, Bit 0 = 0 */ nfc_hal_spi_transfer_byte(((reg_addr << 1) & 0x7E) ¦ 0x80); value = nfc_hal_spi_transfer_byte(0x00); /* Clock in data */ nfc_hal_cs_high(); return value; } void nfc_set_bit_mask(uint8_t reg, uint8_t mask) { uint8_t current = nfc_read_reg(reg); nfc_write_reg(reg, current ¦ mask); } void nfc_clear_bit_mask(uint8_t reg, uint8_t mask) { uint8_t current = nfc_read_reg(reg); nfc_write_reg(reg, current & (~mask)); } /* ========================================================================== */ /* RF Field Control & Initialization */ /* ========================================================================== */ void nfc_rf_field_on(void) { uint8_t current = nfc_read_reg(REG_TX_CONTROL); if ((current & 0x03) != 0x03) { /* Set bits 0 and 1 to enable TX1 and TX2 push-pull drivers */ nfc_write_reg(REG_TX_CONTROL, current ¦ 0x03); /* MANDATORY ISO 14443-3 Section 5.1 Guard Delay */ nfc_hal_delay_ms(5); } } void nfc_rf_field_off(void) { nfc_clear_bit_mask(REG_TX_CONTROL, 0x03); } void nfc_init(void) { nfc_hal_init(); /* Hard reset cycle */ nfc_hal_reset_pin(true); nfc_hal_delay_ms(2); nfc_hal_reset_pin(false); nfc_hal_delay_ms(10); /* Soft reset via SPI command */ nfc_write_reg(REG_COMMAND, CMD_SOFT_RESET); nfc_hal_delay_ms(50); /* Configure internal timers for ISO 14443A timeout detection */ nfc_write_reg(REG_T_MODE, 0x8D); nfc_write_reg(REG_T_PRESCALER, 0x3E); nfc_write_reg(REG_T_RELOAD_HIGH, 0x00); nfc_write_reg(REG_T_RELOAD_LOW, 0x1E); /* Force 100% ASK modulation */ nfc_write_reg(REG_TX_ASK, 0x40); /* Set Mode register to standard 6363h CRC coprocessor */ nfc_write_reg(REG_MODE, 0x3D); /* Turn on RF carrier */ nfc_rf_field_on(); } /* ========================================================================== */ /* ISO/IEC 14443-A CRC-16 Calculation (Polynomial 0x1021, Preset 0x6363) */ /* ========================================================================== */ void nfc_calculate_crc_a(const uint8_t *data, size_t length, uint8_t *crc_out) { uint32_t wcrc = 0x6363; /* ISO 14443-A standard initial seed */ for (size_t i = 0; i < length; i++) { uint8_t byte = data[i]; byte ^= (uint8_t)(wcrc & 0x00FF); byte ^= (byte << 4); wcrc = (wcrc >> 8) ^ ((uint32_t)byte << 8) ^ ((uint32_t)byte << 3) ^ ((uint32_t)byte >> 4); } crc_out[0] = (uint8_t)(wcrc & 0xFF); /* Low byte first */ crc_out[1] = (uint8_t)((wcrc >> 8) & 0xFF); /* High byte second */ } /* ========================================================================== */ /* Transceive Engine: Sends FIFO, waits for tag load-modulation response */ /* ========================================================================== */ nfc_status_t nfc_transceive(const uint8_t *send_data, size_t send_len, uint8_t *recv_data, size_t *recv_len, uint8_t *valid_bits, uint8_t rx_align) { uint8_t wait_irq = 0x30; /* RxIRQ and IdleIRQ */ uint8_t bit_framing = (rx_align << 4) ¦ (valid_bits ? *valid_bits : 0); nfc_write_reg(REG_COMMAND, CMD_IDLE); nfc_write_reg(REG_COMIRQ, 0x7F); /* Clear all pending interrupts */ nfc_set_bit_mask(REG_FIFO_LEVEL, 0x80); /* Flush FIFO */ /* Write payload to FIFO */ for (size_t i = 0; i < send_len; i++) { nfc_write_reg(REG_FIFO_DATA, send_data[i]); } nfc_write_reg(REG_BIT_FRAMING, bit_framing); nfc_write_reg(REG_COMMAND, CMD_TRANSCEIVE); nfc_set_bit_mask(REG_BIT_FRAMING, 0x80); /* Set StartSend bit */ /* Await completion or timeout (max 25 ms) */ uint32_t timeout_counter = 2000; uint8_t irq_val = 0; while (timeout_counter--) { irq_val = nfc_read_reg(REG_COMIRQ); if (irq_val & wait_irq) break; if (irq_val & 0x01) return NFC_STATUS_TIMEOUT; /* TimerIRQ */ nfc_hal_delay_us(10); } if (timeout_counter == 0) return NFC_STATUS_TIMEOUT; /* Check for hardware collision or error flags */ uint8_t err_val = nfc_read_reg(REG_ERROR); if (err_val & 0x08) return NFC_STATUS_COLLISION; /* CollErr */ if (err_val & 0x04) return NFC_STATUS_CRC_ERROR; /* Parity / CRC */ if (err_val & 0x10) return NFC_STATUS_FIFO_OVERFLOW; /* BufferOvfl */ /* Read received response from FIFO */ uint8_t bytes_in_fifo = nfc_read_reg(REG_FIFO_LEVEL); if (recv_len) { if (bytes_in_fifo > *recv_len) bytes_in_fifo = (uint8_t)*recv_len; *recv_len = bytes_in_fifo; for (uint8_t i = 0; i < bytes_in_fifo; i++) { recv_data[i] = nfc_read_reg(REG_FIFO_DATA); } } if (valid_bits) { *valid_bits = nfc_read_reg(REG_CONTROL) & 0x07; } return NFC_STATUS_OK; } /* ========================================================================== */ /* REQA / WUPA Polling: Transmits 7-Bit Short Frame */ /* ========================================================================== */ nfc_status_t nfc_poll_reqa(uint8_t *atqa_out) { uint8_t reqa_cmd = 0x26; /* REQA opcode */ size_t rx_len = 2; uint8_t valid_bits = 7; /* MANDATORY: exactly 7 bits */ /* Clear parity/CRC bits for short frames */ nfc_clear_bit_mask(REG_COLL, 0x80); nfc_status_t status = nfc_transceive(&reqa_cmd, 1, atqa_out, &rx_len, &valid_bits, 0); if (status != NFC_STATUS_OK) return status; if (rx_len != 2 ¦¦ valid_bits != 0) return NFC_STATUS_PROTOCOL_ERROR; return NFC_STATUS_OK; } /* ========================================================================== */ /* Complete Cascade Anti-Collision & Selection Loop */ /* ========================================================================== */ nfc_status_t nfc_select_tag(nfc_tag_t *tag) { uint8_t atqa[2]; nfc_status_t status = nfc_poll_reqa(atqa); if (status != NFC_STATUS_OK) return status; uint8_t cascade_levels[] = {0x93, 0x95, 0x97}; uint8_t uid_offset = 0; for (uint8_t cl = 0; cl < 3; cl++) { uint8_t sel_cmd = cascade_levels[cl]; uint8_t buffer[9]; buffer[0] = sel_cmd; buffer[1] = 0x20; /* NVB = 0x20 (2 bytes known: SEL + NVB) */ size_t rx_len = sizeof(buffer) - 2; status = nfc_transceive(buffer, 2, &buffer[2], &rx_len, NULL, 0); if (status != NFC_STATUS_OK) return status; /* Verify BCC (Block Check Character = XOR of 4 bytes) */ uint8_t bcc = buffer[2] ^ buffer[3] ^ buffer[4] ^ buffer[5]; if (bcc != buffer[6]) return NFC_STATUS_CRC_ERROR; /* Select card (NVB = 0x70) */ buffer[1] = 0x70; nfc_calculate_crc_a(buffer, 7, &buffer[7]); /* Append 2-byte CRC_A */ uint8_t sak_resp[3]; rx_len = sizeof(sak_resp); status = nfc_transceive(buffer, 9, sak_resp, &rx_len, NULL, 0); if (status != NFC_STATUS_OK) return status; tag->sak = sak_resp[0]; /* Check if UID contains Cascade Tag (0x88) */ if (buffer[2] == 0x88) { /* Double or Triple UID: copy 3 real UID bytes, advance to next cascade */ memcpy(&tag->uid_bytes[uid_offset], &buffer[3], 3); uid_offset += 3; } else { /* Final cascade level: copy 4 UID bytes */ memcpy(&tag->uid_bytes[uid_offset], &buffer[2], 4); uid_offset += 4; tag->uid_length = uid_offset; break; } } return NFC_STATUS_OK; }

3. Linux Kernel Subsystem & Driver Integration

In embedded Linux environments (e.g. NXP i.MX8, TI AM62x, Raspberry Pi CM4), NFC transceivers can be integrated via the native Linux NFC Subsystem (`net/nfc`) or through `spidev` with user-space libraries.

+-----------------------------------------------------------------------------------+ ¦ User-Space Applications ¦ ¦ (neard daemon, nfctool, libnfc, Android Open Source Project) ¦ +-----------------------------------------------------------------------------------+ ¦ [AF_NFC / Netlink Sockets] ¦ +-----------------------------------------------------------------------------------+ ¦ Linux Kernel NFC Core (net/nfc) ¦ ¦ (Digital Layer, LLCP Engine, NCI Core, Raw Sockets) ¦ +-----------------------------------------------------------------------------------+ ¦ +-----------------------------------------------------------------------------------+ ¦ Kernel Transceiver Drivers (drivers/nfc/) ¦ ¦ (pn533_spi.c, st25r3916_spi.c, trf7970a.c, nxp-pn544.c) ¦ +-----------------------------------------------------------------------------------+ ¦ [Linux SPI Bus Framework] ¦ +-----------------------------------------------------------------------------------+ ¦ SoC Hardware SPI Controller ¦ +-----------------------------------------------------------------------------------+

3.1 Device Tree Configuration (DTS)

To bind an SPI-based NFC transceiver (e.g. PN532 or ST25R3916) in the Linux Device Tree:

&spi1 { status = "okay"; pinctrl-names = "default"; pinctrl-0 = <&pinctrl_spi1>; nfc@0 { compatible = "nxp,pn532"; reg = <0>; /* Chip Select index (CS0) */ spi-max-frequency = <5000000>; /* 5 MHz max clock */ interrupt-parent = <&gpio1>; interrupts = <18 IRQ_TYPE_EDGE_FALLING>; reset-gpios = <&gpio1 19 GPIO_ACTIVE_LOW>; /* Optional: Power-down GPIO */ enable-gpios = <&gpio1 20 GPIO_ACTIVE_HIGH>; status = "okay"; }; };

3.2 User-Space libnfc Integration over SPI

If using libnfc via user-space /dev/spidev0.0, configure /etc/nfc/devices.d/pn532_spi.conf:

name = "PN532 over SPI" connstring = "pn532_spi:/dev/spidev0.0:5000000" allow_intrusive_scan = true

Test hardware bring-up from the Linux shell:

# Scan for connected NFC transceivers nfc-scan-device -v # Continuously poll for tags and print UID/SAK/ATQA nfc-poll

4. Hardware Bring-Up & Signal Integrity Verification

Bring-up must follow a structured 4-step physical verification process:

+-------------------+ +--------------------+ +--------------------+ +--------------------+ ¦ STEP 1: SPI BUS ¦ ¦ STEP 2: RF CARRIER ¦ ¦ STEP 3: MODULATION ¦ ¦ STEP 4: CARD LOAD ¦ ¦ - Verify SCLK/CS ¦ ===> ¦ - Probe 13.56 MHz ¦ ===> ¦ - Verify 100% ASK ¦ ===> ¦ MODULATION ¦ ¦ - Read Reg 0x37 ¦ ¦ - Sine Vpp 10-20V ¦ ¦ - Pause t1: 2-3 us ¦ ¦ - Probe 848 kHz ¦ ¦ (Chip Version) ¦ ¦ - Check harmonics ¦ ¦ - Check overshoot ¦ ¦ sidebands on spy ¦ +-------------------+ +--------------------+ +--------------------+ +--------------------+

Step 1: SPI Bus Verification

Attach logic analyzer channels to SCLK, MOSI, MISO, CS#, and IRQ.

Issue a read of the silicon version register (Address 0x37 on MFRC522 \rightarrow Expected value: 0x92 for v2.0 or 0x91 for v1.0).

If MISO returns 0x00 or 0xFF: Check clock polarity (CPOL=0), chip select assertion, and verify power rails.

Step 2: RF Carrier Verification

Attach a high-impedance 10:1 passive oscilloscope probe across TX1 and TX2 (or across the antenna coil).

Verify continuous 13.560 MHz ($\pm 7\text{ kHz}$) sinusoidal oscillation.

Confirm differential amplitude: V_{pp across the coil should be between $10\text{V}$ and $20\text{V}$ (or 3.3V single-ended).

Step 3: Modulation Envelope Verification

Capture the RF carrier while transmitting a REQA command (0x26).

Measure the carrier pause width: must fall strictly within $2.0\ \mu\text{s} \le t_1 \le 3.0\ \mu\text{s}$.

If excessive ringing or overshoot is observed after the pause, the antenna Q-factor is too high; increase the value of series damping resistors (R_Q).

Step 4: Tag Load Modulation Verification

Place a 2-turn pickup coil ("spy coil") connected to an oscilloscope near the antenna.

Approach the reader with an NTAG213 or MIFARE card.

Zoom in on the RF envelope after the REQA pause. Look for the card's 848 kHz subcarrier burst (12.712 MHz / 14.408 MHz sideband modulation) exhibiting Manchester transitions.

5. Troubleshooting & Root Cause Analysis (RCA) Matrix

| Symptom | Probable Root Cause | Diagnostic Check | Verified Engineering Fix |
| --- | --- | --- | --- |
| Transceiver unresponsive; reads return 0x00 or 0xFF | SPI mode mismatch or floating reset pin | Check MISO on scope; probe \overline{RSTPD pin | Configure SPI Controller for Mode 0 (CPOL=0, CPHA=0). Ensure \overline{RSTPD is driven HIGH (3.3V). |
| PN532 commands return no ACK or garbled response | Bit order mismatch (LSB-First trap) | Inspect MOSI data on logic analyzer | PN532 hardware SPI requires LSB-first transmission. Reverse bit order in software or enable LSB-first in MCU SPI hardware. |
| Card only detected at zero distance (< 3 mm) | Antenna detuning or mismatched Q-factor | Measure antenna inductance L_{ant with LCR meter | Adjust matching capacitors (C_{series, C_{parallel) to center resonance at 13.56 MHz; ensure Q \approx 20--30. |
| RF carrier turns on, but tags never respond to REQA | Omission of $5.0\text{ ms}$ guard delay ($T_{guard}$) | Scope probe RF carrier vs REQA burst | Insert delay_ms(5) immediately after enabling TX_CONTROL before sending REQA to allow tag reservoir capacitors to charge. |
| Microcontroller crashes or resets when RF field enables | Inrush current causing VDD rail sag | Monitor 3.3V power rail with scope trigger during RF turn-on | Add bulk decoupling capacitor (10\ µF tantalum or low-ESR ceramic) adjacent to transceiver TVDD and AVDD pins. |
| Card detected, but anti-collision fails (`CollErr` asserted) | High RF noise or receiver gain saturated | Read RFCfgReg and CollReg | Lower the internal receiver gain setting (e.g. from 48 dB to 38 dB); adjust RxThreshold register. |
| Tags completely undetected when mounted on metal/battery | Eddy currents in metal canceling magnetic flux | Probe H-field strength near metal surface | Apply a sintered ferrite shielding sheet (µ_r' ≥ 100) between the PCB loop antenna and the metal surface. |
| CRC errors during high-speed transfers (424 / 848 kbps) | Antenna bandwidth too narrow (Q too high) | Calculate BW = f_c / Q | Increase damping resistor R_Q to reduce Q to 10--15, widening RF bandwidth to > 1 MHz. |