REACH Compliance: Architecture, Regulatory Obligations, and Electronics Hardware Guide

1. What is REACH?

REACH stands for Registration, Evaluation, Authorisation, and Restriction of Chemicals. It is a cornerstone regulation of the European Union—Regulation (EC) No 1907/2006—overseen by the ECHA (European Chemicals Agency) based in Helsinki, Finland.

Unlike RoHS, which is an industry-specific directive targeting electrical and electronic equipment, REACH is an overarching horizontal regulation encompassing all chemical substances across all industries, whether used in industrial chemical processes, daily consumer goods, paints, textiles, or advanced electronic assemblies.

The Four Pillars of REACH:

┌─────────────────────────────────────────────────────────────────────────────┐ │ THE 4 PILLARS OF REACH │ ├─────────────────────────────────────────────────────────────────────────────┤ │ 1. REGISTRATION Manufacturers & importers of raw chemical substances │ │ must register chemical dossiers with ECHA (≥ 1 tonne/yr).│ ├─────────────────────────────────────────────────────────────────────────────┤ │ 2. EVALUATION ECHA and EU Member States evaluate dossiers to assess │ │ health, aquatic toxicity, and environmental risks. │ ├─────────────────────────────────────────────────────────────────────────────┤ │ 3. AUTHORISATION Substances of Very High Concern (SVHC) listed on Annex │ │ XIV are phased out unless specific authorization is won. │ ├─────────────────────────────────────────────────────────────────────────────┤ │ 4. RESTRICTION Annex XVII prohibits or strictly limits the manufacture, │ │ marketing, or use of specific dangerous substances. │ └─────────────────────────────────────────────────────────────────────────────┘

2. REACH Terminology for Hardware Engineers: Substances, Mixtures, and Articles

Understanding REACH requires distinguishing between three regulatory definitions established in Article 3 of the regulation:

| Term | Legal Definition (Article 3) | Examples in Hardware Engineering | REACH Regulatory Focus |
| --- | --- | --- | --- |
| Substance | A chemical element and its compounds in the natural state or obtained by any manufacturing process. | Pure Copper (Cu), pure Gold (Au), Lead metal (Pb), Silicon (Si). | Substance registration (Article 6). |
| Mixture / Preparation | A mixture or solution composed of two or more substances. | Solder paste, liquid flux, adhesive epoxies, conformal coatings, thermal greases. | Safety Data Sheets (SDS), volume registration. |
| Article | An object which during production is given a special shape, surface, or design which determines its function to a greater degree than does its chemical composition. | Finished IC packages, printed circuit boards (PCBs), connectors, resistors, screws, cables, chassis enclosures, complete PCBAs. | Article 33 Duty to Communicate; SCIP reporting; Annex XVII restrictions. |

[NOTE] Electronic systems integrators and ODM/EMS providers like VVDN primarily design, assemble, and import Articles and Complex Objects (assemblies made up of multiple individual articles).

3. SVHC (Substances of Very High Concern) & The Candidate List

A substance is classified as a Substance of Very High Concern (SVHC) if it meets the rigorous hazard criteria defined in Article 57 of REACH:

CMR: Carcinogenic, Mutagenic, or Toxic for Reproduction (Category 1A or 1B).

PBT: Persistent, Bioaccumulative, and Toxic (per Annex XIII criteria).

vPvB: Very Persistent and Very Bioaccumulative.

Equivalent Level of Concern: Substances such as endocrine disruptors that cause serious effects on humans or ecosystems equivalent to CMR or PBT.

The Candidate List Update Dynamic

When an SVHC is proposed and approved, it is placed on the ECHA Candidate List for Authorisation.

Update Frequency: The Candidate List is updated twice every year (typically in January and June/July).

Living Document: The list has grown continuously from 15 substances in 2008 to over 240+ substances.

Immediate Effect: The legal obligations for an article manufacturer/importer trigger on the exact day a substance is published on the Candidate List. There is no multi-year transition grace period.

Common SVHCs Encountered in Hardware & Electronics

| Substance Name | CAS Number | Why It Appears in Electronics Hardware |
| --- | --- | --- |
| Lead Metal ($Pb$) | 7439-92-1 | Added to Candidate List in June 2018. Present in brass connectors (up to 4% Pb under RoHS exemption 6c), high-temp solders (RoHS exemption 7a). |
| Lead Monoxide ($PbO$) / Lead Dioxide ($PbO_2$) | 1317-36-8 | Constituent of glass passivations in thick-film SMD resistors and diodes (RoHS exemption 7c-I). |
| Lead Titanium Zirconium Oxide (PZT) | 12626-81-2 | Piezoelectric buzzers, ultrasound transducers, ceramic resonators. |
| Octamethylcyclotetrasiloxane (D4) / Decamethylcyclopentasiloxane (D5) | 556-67-2 / 541-02-6 | Residual monomer impurities in silicone thermal pads, silicone gaskets, and potting compounds. |
| 2-Benzyl-2-dimethylamino-4'-morpholinobutyrophenone | 119313-12-1 | Photoinitiator used in UV-curable solder masks and photoresists. |
| Dechlorane Plus | 13560-89-9 | Flame retardant used in cable insulation, electrical tape, and structural plastics. |
| Melamine | 108-78-1 | Flame retardant additive in polymer housings and terminal blocks. |

4. Article 33 Duty to Communicate & Consumer Requests

Article 33 of REACH imposes a statutory obligation throughout the commercial supply chain regarding SVHCs:

4.1 Business-to-Business (B2B) Communication (Article 33(1))

If any article supplied contains an SVHC in a concentration exceeding $0.1\%$ weight-by-weight ($w/w$), the supplier must automatically provide the recipient with:

The name of the SVHC present.

Sufficient information available to ensure the safe handling and use of the article (e.g., disposal guidelines, skin contact warnings).

4.2 Business-to-Consumer (B2C) Communication (Article 33(2))

Any consumer can ask a retailer or manufacturer whether an article contains an SVHC > 0.1\%\,w/w.

The supplier is legally bound to provide this information free of charge within 45 calendar days of the request.

5. The Landmark ECJ Ruling: "Once an Article, Always an Article" (O5A)

Historically, some manufacturers interpreted the 0.1\%\,w/w threshold by calculating the weight of the SVHC against the total weight of the entire finished product (e.g., dividing the lead in a micro-resistor by the weight of an entire 5 kg server).

On September 10, 2015, the European Court of Justice (ECJ) delivered a landmark ruling in Case C-106/14:

[NOTE] "Once an Article, Always an Article" (O5A): An article remains an article as long as it retains its shape, surface, or design. When individual articles are assembled or joined together into a "complex object" (such as a PCBA, sub-assembly, or complete device), each individual sub-article retains its independent duty of notification.

Mathematical Worked Example: Why Dilution is Illegal

┌─────────────────────────────────────────────────────────────┐ │ COMPLEX OBJECT: Server Motherboard (Total Weight: 1000.0 g) │ │ │ │ ┌─────────────────────────────────────────────────────┐ │ │ │ SUB-ARTICLE: SMT Thick-Film Resistor │ │ │ │ Component Total Weight: 0.002 g │ │ │ │ Glass Layer contains Lead Monoxide (PbO): 0.0001 g │ │ │ └─────────────────────────────────────────────────────┘ │ └─────────────────────────────────────────────────────────────┘

Incorrect Dilution Calculation (Illegal):

\text{Concentration} = \frac{\text{Mass of PbO}}{\text{Total Motherboard Weight}} = \frac{0.0001\,\text{g}}{1000.0\,\text{g}} = 0.00001\% = 0.1\,\text{ppm} \quad (\ll 0.1\%)

Under this flawed logic, no disclosure would be made.

Correct O5A Legal Calculation (Mandatory):

The resistor is an independent article before assembly into the complex board:

\text{Concentration} = \frac{\text{Mass of PbO}}{\text{Total Resistor Weight}} = \frac{0.0001\,\text{g}}{0.002\,\text{g}} = 5.0\%\,w/w

Since 5.0\% > 0.1\%, Article 33 notification IS MANDATORY. The manufacturer must disclose that the PCBA contains Lead Monoxide in its thick-film chip resistors.

6. Annex XVII (Restrictions) vs Annex XIV (Authorisation)

| Parameter | Annex XVII (Restricted Substances) | Annex XIV (Authorisation List) |
| --- | --- | --- |
| Purpose | Immediate or scheduled ban or strict cap on specific uses where an unacceptable risk to human health or the environment is proven. | Mechanism to encourage industry to phase out SVHCs and replace them with safer alternatives. |
| Enforcement Mechanism | Prohibits placing on the market or using the substance for specified restricted applications. | The substance cannot be used in the EU after a specific "Sunset Date", unless the user holds a specific, approved Authorization granted by the European Commission. |
| Applies to Articles? | YES. Many restrictions specifically govern finished articles (e.g. nickel release on smartwatch cases). | Strictly governs manufacturing operations inside the EU. Imported finished articles do not require Annex XIV authorization (though they remain subject to Article 33, SCIP, and Annex XVII). |
| Key Hardware Examples | - Entry 27 (Nickel): Restricts nickel release (< 0.5\,µg/cm^2/week) on items with prolonged skin contact (wearable fitness bands, smartwatches).<br>- Entry 51/52 (Phthalates): Restricts DEHP, DBP, BBP, DIBP in toys and childcare equipment.<br>- Entry 63 (Lead): Restricts lead in jewelry and articles that children could mouth. | - Hexavalent chromium compounds used in surface plating facilities within the EU.<br>- Certain industrial phthalates used as plasticizers in EU-based polymer processing plants. |

7. SCIP Database Reporting (EU Waste Framework Directive)

Under the revised Waste Framework Directive (Directive (EU) 2018/851), Article 9(1)(i) mandates that any company producing, importing, or distributing articles containing Candidate List SVHCs > 0.1\%\,w/w on the EU market must submit detailed product data to the SCIP Database managed by ECHA.

Mandatory Live Date: January 5, 2021.

Acronym: Substances of Concern In articles as such or in complex objects (Products).

Target Audience: Waste operators, recyclers, and consumers seeking circular economy disassembly data.

Hardware Manufacturer / Importer │ ▼ [Scrub BOM for Articles with SVHC > 0.1% w/w] │ ▼ [Prepare SCIP Dossier (IUCLID Format)] ├── Primary Article Identifier (Part Number / EAN / GTIN) ├── TARIC / CN Customs Code ├── Safe Use Instructions & Disassembly Guidance └── Material Category & Linking to Specific Sub-Articles │ ▼ [Submit via ECHA Submission Portal] │ ▼ Receives Unique SCIP Number (UUID) ──► Shared with downstream EU distributors

8. Detailed Comparison: RoHS vs REACH

Engineers often confuse RoHS and REACH because both are EU environmental regulations covering hazardous chemicals. Their legal architectures, however, differ fundamentally:

| Comparison Dimension | RoHS (Directive 2011/65/EU + 2015/863) | REACH (Regulation (EC) No 1907/2006) |
| --- | --- | --- |
| Legal Nature | Directive: Must be transposed into national laws of each EU Member State. | Regulation: Directly applicable and legally binding in all EU Member States without national transposition. |
| Scope of Products | Specifically Electrical & Electronic Equipment (EEE) operating ≤ 1000\,V AC or ≤ 1500\,V DC. | All physical goods, chemicals, and articles (electronics, clothing, toys, automotive, chemicals). |
| Number of Substances | Exactly 10 restricted substances (Pb, Cd, Hg, Cr6+, PBB, PBDE, and 4 phthalates). | 240+ substances on Candidate List (SVHC), plus hundreds in Annex XVII and Annex XIV. |
| Threshold Calculation Basis | Per Homogeneous Material (e.g., pin plating, solder joint, mold compound). | Per Individual Article under the O5A ruling (e.g., entire resistor, bare screw, connector housing). |
| What Happens if Exceeded? | BANNED. Product cannot be placed on the EU market unless an explicit Annex III/IV exemption applies. | ALLOWED (usually), but triggers mandatory reporting (Article 33 customer disclosure and SCIP database filing). (Annex XVII bans specific uses). |
| CE Marking | MANDATORY. RoHS is a direct CE marking directive; product requires an EU Declaration of Conformity (DoC). | NO CE MARK. REACH does not grant or govern the CE mark. |
| Exemptions Mechanism | Fixed list in Annex III and IV with defined expiration dates. | Authorisation (Annex XIV) or socio-economic derogations. |

9. Hardware Engineering Compliance Workflow at VVDN

To ensure continuous REACH compliance during product design, component procurement, and manufacturing:

Bi-Annual Candidate List Audits:

Establish automated alerts whenever ECHA updates the SVHC list (every January and June/July).

Re-screen existing Bills of Materials (BOMs) against newly added substance CAS numbers.

Supply Chain Data Collection:

Request Full Material Disclosure (FMD) in IPC-1752A Class D XML format from semiconductor vendors.

For custom passive, plastic, or mechanical parts, obtain signed REACH SVHC Declarations specifying the active Candidate List version.

Complex Object Hierarchy Modeling:

Maintain product assembly trees in PLM/ERP software to track which specific sub-tier articles contain SVHCs (e.g., brass standoffs containing lead > 0.1\%\,w/w).

SCIP Dossier Generation:

Before shipping products to European customers, generate the ECHA IUCLID dossier and file the SCIP submission.

Transmit the resulting SCIP UUID to European distributors and sales channels.