N-Channel Enhancement-Mode MOSFET (e-NMOS): Physics, Design & Applications

An N-Channel Enhancement-Mode MOSFET (commonly designated e-NMOS) is the most ubiquitous discrete transistor and integrated circuit building block in electronics. Being a normally-off device, no conduction channel exists at zero gate bias (V_{GS = 0\,V). It relies on an applied positive gate potential (V_{GS > V_{TH) to electrostatically induce an electron inversion layer in a p-type substrate.

1. Semiconductor Physics & Device Structure

Gate (G) │ ┌─────┴─────┐ Poly-Si / Metal Gate │ GATE │ ┌────┴───────────┴────┐ │ Gate Oxide (SiO2) │ (tox: 10nm - 50nm) ┌───────────────┴─────────────────────┴───────────────┐ │ Source (n+) Drain (n+) │ ───┤ ┌───────┐ Inversion Channel ┌───────┐ ├─── (S) │ │ n+ │ ========================= │ n+ │ │ (D) └──┴───────┴─────────────────────────────┴───────┴────┘ │ Depletion Layer (Wdep) │ │ - - - - - - - - - - - - - - - - - - - - - - - - - │ │ p-type Substrate │ │ │ └──────────────────────────┬──────────────────────────┘ │ Body (B) (Internally tied to Source)

Physical Working Steps:

Zero Gate Bias ($V_{GS} = 0\,\text{V}$):

The n^+ Source, p Substrate, and n^+ Drain form two back-to-back p-n diodes (n^+-p and p-n^+).

Any voltage applied across Drain-to-Source (V_{DS > 0) reverse-biases the Drain-to-Body junction. Only tiny reverse leakage current (I_{DSS \approx nA) flows.

Threshold Formation ($V_{GS} = V_{TH}$):

A positive voltage on the gate creates a vertical downward electric field (E_{vert).

Mobile holes (h^+) in the p-substrate are repelled away from the Si-SiO_2 interface, uncovering negatively charged immobile acceptor ions (N_A^-) to form a depletion region.

When V_{GS reaches the Threshold Voltage ($V_{TH} \approx 1.5\,\text{V} - 4.0\,\text{V}$), the conduction band bends below the Fermi level at the surface, pulling in free minority electrons (e^-). This forms an n-type surface inversion channel.

Channel Conduction ($V_{GS} > V_{TH}$):

Because electrons are the majority carriers in the channel, they possess high drift mobility (µ_n \approx 1350 - 1450\,cm^2/V\cdots in bulk Silicon), enabling very low on-resistance per unit die area compared to PMOS.

2. Detailed Characteristic Curves & Working Region Waveforms

The N-channel enhancement MOSFET operates across distinct physical regimes depending on gate-source voltage (V_{GS) and drain-source voltage (V_{DS).

2.1 Static Output Characteristics Waveform (I_D vs. V_{DS)

Drain Current (Id) vs. Drain-to-Source Voltage (Vds) Id ^ │ Vgs = 10V (Deep Triode & Saturation) │ .─────────────────────────── / / ───┐ Avalanche │ .───' │ Breakdown │ .───' Vgs = 8V │ (V_BR) │ .───' .───────────────────────── / / ───┤ │ .───' .───' │ │ .───' .───' Vgs = 6V │ │ .───' .───' .───────────────────────── / / ───┤ │ .───' .───' .───' │ │ .───' .───' .───' Vgs = 4V │ │ .───' .───' .───' .───────────────────────── / / ───┤ │ / .───' .───' .───' │ │ / .───' .───' .───' Vgs = 2.5V (Near Threshold) │ │/ .───' .───' .───' .───────────────────────── / / ───┤ │ .──' .───' .───' .───' │ │/ .───' .───' .───' │ │ .───' .───' .───' Vgs = 2.0V = Vth │ │.───' .───' .───' .───────────────────────── / / ───┤ ├────'──────────'───────────'───────────'─────────────── Vgs < Vth (Cutoff: Id ~ 0)──┴─────── │ ◄───────────► │ ◄─────────────────────────────────────────────► │ ◄──────────────────────► 0 Linear / Pinch-Off Boundary Saturation / Active Avalanche Breakdown Triode Region Vds = Vgs - Vth Region Vds >= V_BR(DSS) (Vds < Vgs-Vth) (Channel collapsed (Vds >= Vgs-Vth) (Impact Ionization) Rds_on = Vds/Id at Drain) Id = 0.5*kn*(Vgs-Vth)^2

Working Region Operating Equations:

Cutoff Region ($V_{GS} < V_{TH}$):

Inversion channel is absent; only subthreshold diffusion leakage flows:

I_D \approx I_{0} \cdot \exp\left(\frac{q(V_{GS} - V_{TH})}{n k T}\right) \cdot \left[1 - \exp\left(-\frac{q V_{DS}}{k T}\right)\right] \approx 0

Linear / Triode / Ohmic Region ($V_{GS} > V_{TH}$ and $V_{DS} < V_{GS} - V_{TH}$):

Electron channel connects Source to Drain continuously:

I_D = \mu_n C_{ox} \left(\frac{W}{L}\right) \left[ (V_{GS} - V_{TH}) V_{DS} - \frac{1}{2} V_{DS}^2 \right]

For very low V_{DS \ll 2(V_{GS - V_{TH), the channel acts as an ideal resistor R_{DS(on):

R_{DS(on)} = \frac{1}{\mu_n C_{ox} (W/L) (V_{GS} - V_{TH})}

Pinch-Off Point ($V_{DS} = V_{GS} - V_{TH}$):

Local surface inversion depth at the drain corner collapses to zero.

Saturation (Active) Region ($V_{GS} > V_{TH}$ and $V_{DS} \ge V_{GS} - V_{TH}$):

Pinch-off point moves slightly toward the source; current is constrained by carrier drift:

I_{D(sat)} = \frac{1}{2} \mu_n C_{ox} \left(\frac{W}{L}\right) (V_{GS} - V_{TH})^2 (1 + \lambda V_{DS})

Small-signal transconductance: g_m = \sqrt{2 µ_n C_{ox (W/L) I_D = \frac{2 I_D{V_{GS - V_{TH.

Avalanche Breakdown Region ($V_{DS} \ge V_{BR(DSS)}$):

Carrier multiplication due to high reverse field across the drain-body junction.

2.2 Transfer Characteristics Waveform (I_D vs. V_{GS)

Linear Scale (Square-Law Conduction) Logarithmic Scale (Subthreshold Swing) Id ^ log(Id)^ │ Saturation Regime │ Strong Inversion │ Id ~ (Vgs - Vth)^2 10^-2 ┼ (Ohmic / Saturation) │ .─' │ / │ .─' 10^-4 ┼ / │ .─' │ / │ .─' 10^-6 ┼ / │ .─' │ / │ .─' 10^-8 ┼ / Subthreshold Swing: │ .─' │ / S = dVgs / d(log10 Id) │ .─' │ / ~ 70mV/decade │ .─' 10^-10 ┼ / │ .─' │ / │ .─' Subthreshold Exponential Leakage 10^-12 ┼────────/ <-- Off-state leakage floor └─────┴────────────────────────────────> Vgs └────────┴─────────────────────────────> Vgs 0 Vth 0 Vth

2.3 Third-Quadrant Reverse Conduction Waveform (V_{DS < 0\,V)

Quadrant III Conduction: Body Diode vs. Synchronous Rectification -Id (Reverse Current: Source to Drain) ^ │ Active Synchronous Rectifier (Vgs = 10V) │ Linear Ohmic Conduction: -Vds = -Id * Rds_on (< 30mV) │ .─' │ .─' │ .─' │ .─' │ .─' Passive Body Diode Conduction (Vgs = 0V) │ .─' Standard p-n Diode Knee: VF ~ 0.7V - 1.2V │ / High forward dissipation & Qrr recovery! │ / │/ ├───────────────────────. │ \ │ \ │ \ -Vds ◄─┴───────────────────────────┴────── 0 (Drain-to-Source Voltage) -VF (~0.8V)

2.4 Time-Domain Inductive Switching Waveform & Regional Trajectory

When an e-NMOS switches an inductive load (solenoid, motor, or buck stage), it transitions sequentially across all operating regions:

Time-Domain Switching Waveform & Region Trajectory │◄─ t_d(on) ─►│◄─ t_ri ─►│◄── t_vf ──►│◄─ t_enh ─►│ Vgs ^ │ │ │ │ │ │ │ │ │Miller Plat.│ V_drive │ V_drv┼─┼─────────────┼──────────┼────────────┼──.────────┤ V_plt┼─┼─────────────┼──────────┼────────────┴'──────────┤ V_th┼─┼─────────────┼──────────/ │ │ 0V ┴─┴─────────────┴─────────'┴────────────┴───────────┴──> Time │ │ │ │ │ Vds ^ │ │ │ │ │ V_bus┼─┼─────────────┼──────────┼────────────┼───────────┤ │ │ │ │\ │ │ │ │ │ │ \ │ Rds*Id │ 0V ┴─┴─────────────┴──────────┴──'─────────┴──.────────┴──> Time │ │ │ │ │ Id ^ │ │ │ │ │ I_load┼─┼─────────────┼──────────┼────────────┼───────────┤ │ │ │ │ │ │ │ │ │/ │ │ │ 0A ┴─┴─────────────'──────────┴────────────┴───────────┴──> Time ────────┼─────────────┼──────────┼────────────┼───────────┤ REGION: │ 1. CUTOFF │ 2.ACTIVE │ 3.ACTIVE-> │ 4. LINEAR │ │ │(SATURAT.)│ LINEAR │ (OHMIC) │ ────────┴─────────────┴──────────┴────────────┴───────────┘

Turn-on Delay ($t_{d(on)}$): Device is in Cutoff (V_{GS < V_{TH). I_D = 0, V_{DS = V_{BUS.

Current Rise ($t_{ri}$): Device enters Saturation (V_{GS > V_{TH, V_{DS ≥ V_{GS - V_{TH). I_D ramps up to load current while V_{DS remains clamped at V_{BUS.

Voltage Fall / Miller Plateau ($t_{vf}$): Device traverses from Saturation toward Linear boundary. Gate voltage is held at V_{plateau while C_{gd is discharged and V_{DS collapses.

Channel Enhancement ($t_{enh}$): Device enters Deep Linear (Ohmic) regime. V_{GS rises to V_{DRIVE, settling R_{DS(on) to minimum.

3. Circuit Implementation: Low-Side Switch Architecture

In automotive ECUs, microcontroller peripherals, and motor drivers, the e-NMOS is the gold standard for low-side switching:

+V_LOAD (+12V / +24V Rail) │ ├───┐ │ │ [ LOAD ] (Solenoid / Relay / Motor / LED string) │ │ ├───┴─── [Freewheeling Diode: 1N4007 / SS34] │ Drain (D) │ ┌──┴──┐ R_gate │ │ MCU ───[ 100Ω ]───┤ NMOS│ GPIO │ │ ┌────┴──┬───┘ │ │ [10k] │ Source (S) R_pd │ │ │ GND GND (Common Ground Return)

Hardware Design Rules:

Pull-Down Resistor ($R_{pd} \approx 10\,\text{k}\Omega - 100\,\text{k}\Omega$):

Placed directly between Gate and Source.

Prevents the high-impedance gate from floating during MCU reset, bootloader execution, or high-Z uninitialized GPIO states, which would cause parasitic turn-on and burn out the transistor.

Series Gate Resistor ($R_{gate} \approx 10\,\Omega - 100\,\Omega$):

Damps LC ringing formed between trace parasitic inductance (L_{gate) and MOSFET input capacitance (C_{iss).

Limits the peak transient sourcing/sinking current pulled from the MCU GPIO.

Freewheeling Diode:

Required for inductive loads. When the NMOS abruptly turns off, inductor current cannot instantaneously drop to zero (\Delta V = -L \frac{di{dt), causing a positive voltage spike at the Drain that will exceed V_{(BR)DSS and destroy the device if not clamped.

4. Key Datasheet Parameters & Electrical Traps

| Datasheet Parameter | Symbol | Critical Significance | Design Rule of Thumb |
| --- | --- | --- | --- |
| Drain-Source Breakdown Voltage | V_{(BR)DSS | Maximum voltage across D-S before avalanche occurs | Select with ≥ 20\% - 50\% margin above supply bus |
| Gate-Source Voltage Limit | V_{GS(max) | Dielectric breakdown rating of thin SiO_2 (typically ± 20\,V) | Clamp with 15\,V - 18\,V TVS / Zener if transients exist |
| Threshold Voltage | V_{GS(th) | Gate voltage where I_D \approx 250\,µA begins conduction | Must NOT be confused with fully enhanced drive voltage (V_{GS ≥ 10\,V) |
| Static Drain-Source On-Resistance | R_{DS(on) | Internal resistance when fully ON at specified V_{GS and T_j | Derate by 1.5× - 2.0× for T_j = 125^\circC operation |
| Total Gate Charge | Q_g | Total charge needed to raise V_{GS to operating level | Determines driver current requirements: I_{drive = Q_g / t_{target |
| Reverse Recovery Charge | Q_{rr | Body diode recovery charge during commutation | Causes shoot-through and ringing in half-bridge converters |

5. Commercial Part Catalog & Selection

| Part Number | Manufacturer | Package | V_{DS(max) | I_{D(max) | R_{DS(on) (@10V) | Typical Use-Case |
| --- | --- | --- | --- | --- | --- | --- |
| 2N7002 | onsemi / Diodes Inc | SOT-23 | 60\,V | 300\,mA | 5.0\,\Omega | Small-signal level shifting, LED drive, logic gates |
| BSS138 | onsemi / Fairchild | SOT-23 | 50\,V | 220\,mA | 3.5\,\Omega | I^2C bidirectional level translator |
| IRLZ44N | Infineon (IR) | TO-220 | 55\,V | 47\,A | 22\,m\Omega | 5V Logic-level hobbyist/industrial load switching |
| IRF540N | Infineon (IR) | TO-220 | 100\,V | 33\,A | 44\,m\Omega | Classical power switching, audio amplifier, solenoids |
| BSC030N04NS | Infineon | TDSON-8 (SuperSO8) | 40\,V | 100\,A | 3.0\,m\Omega | Automotive synchronous buck, high-efficiency PoL |