DC-DC Converters: Isolated & Non-Isolated Topologies Master Guide

Welcome to the VVDN Engineering Hub DC-DC Converters Knowledge Base. This module provides an exhaustive architectural, analytical, and design guide covering all primary Isolated and Non-Isolated switched-mode DC-DC power conversion topologies.

1. DC-DC Taxonomy & Classification Tree

DC-DC Converter Topologies │ ┌─────────────────────────────────────────┴─────────────────────────────────────────┐ ▼ ▼ Isolated Topologies (Transformer Isolation) Non-Isolated Topologies (Direct Inductor) ├── 1. Forward Converter (Tertiary / 2-Switch Reset) ├── 1. Buck Converter (Step-Down, Vout < Vin) ├── 2. Flyback Converter (Coupled Inductor, DCM/CCM) ├── 2. Boost Converter (Step-Up, Vout > Vin) ├── 3. Dual Active Bridge (DAB: Bidirectional Phase-Shift) ├── 3. Buck-Boost Converter (Inverting / 4-Switch) ├── 4. Full-Bridge Converter (Phase-Shifted ZVS / PSFB) ├── 4. Cuk Converter (Capacitive Transfer, Low EMI) ├── 5. Half-Bridge Converter (Capacitive Divider Split) ├── 5. SEPIC Converter (Non-Inverting Buck-Boost) ├── 6. Push-Pull Converter (Center-Tapped Primary) └── 6. Zeta Converter (Inverse SEPIC, Cont. Iout) └── 7. Resonant Converters [HIGH IMPORTANCE] ├── Series Resonant Converter (SRC) ├── Parallel Resonant Converter (PRC) └── LLC Resonant Half-Bridge Converter (ZVS / ZCS)

2. Comprehensive Topology Benchmark & Comparison Matrix

| Topology | Isolation? | Voltage Gain (\frac{V_{OUT{V_{IN) | Power Spectrum | Switch Voltage Stress (V_{sw) | Switch Current Stress (I_{sw) | Magnetic Core Utilization | Target Applications |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Buck | No | D | 1\,W \dots 2\,kW | V_{IN | I_{OUT | Single inductor, DC bias | PoL, CPU VRM, battery chargers |
| Boost | No | \frac{1{1 - D | 5\,W \dots 5\,kW | V_{OUT | \frac{I_{OUT{1 - D | Single inductor, DC bias | PFC pre-regulators, Solar boost, LED |
| Buck-Boost | No | -\frac{D{1 - D | 5\,W \dots 500\,W | V_{IN + V_{OUT | \frac{I_{OUT{1 - D | Single inductor, DC bias | Automotive battery systems, Handhelds |
| Cuk | No | -\frac{D{1 - D | 10\,W \dots 1\,kW | V_{IN + V_{OUT | \frac{I_{OUT{1 - D | Dual inductors (can couple) | Low-noise bipolar rails, audio |
| SEPIC | No | \frac{D{1 - D | 5\,W \dots 500\,W | V_{IN + V_{OUT | \frac{I_{OUT{1 - D | Dual inductors / coupled | Battery-powered devices, LED drivers |
| Zeta | No | \frac{D{1 - D | 5\,W \dots 500\,W | V_{IN + V_{OUT | \frac{I_{OUT{1 - D | Dual inductors / coupled | Telecom low-ripple supplies |
| Flyback | Yes | n \cdot \frac{D{1 - D | 1\,W \dots 150\,W | V_{IN + n V_{OUT | \frac{I_{OUT{n(1 - D) | Unipolar coupled inductor | Standby supplies, AC adapters, Gate bias |
| Forward | Yes | n \cdot D (D < 0.5) | 50\,W \dots 500\,W | 2 V_{IN (1-switch) | \frac{n I_{OUT{D | Unipolar transformer + reset | Industrial telecom bricks, DC-DC isolated |
| Push-Pull | Yes | 2n \cdot D | 100\,W \dots 1.5\,kW | 2 V_{IN | n I_{OUT | Bipolar (Symmetric quadrants) | 12V/24V inverters, automotive DC-DC |
| Half-Bridge | Yes | n \cdot D | 100\,W \dots 1.5\,kW | V_{IN | 2n I_{OUT | Bipolar (Symmetric quadrants) | PC power supplies, server auxiliary |
| Full-Bridge | Yes | 2n \cdot D (PSFB) | 1\,kW \dots 20\,kW | V_{IN | n I_{OUT | Bipolar (Symmetric quadrants) | EV fast charging, industrial welders |
| Dual Active Bridge | Yes | $n \cdot \frac{\phi(\pi - | \phi | )}{\pi^2 f L}$ | 1\,kW \dots 100\,kW | V_{IN / V_{OUT | Continuous AC peak |
| LLC Resonant | Yes | n \cdot M(f_n, Q, k) | 100\,W \dots 10\,kW | V_{IN (ZVS turn-on) | Resonant sinusoidal | Bipolar resonant transformer | Server PSUs, Flat-panel TV, EV OBC |

3. Directory Navigation & Technical Dossiers

Isolated DC-DC Topologies:

**`isolated/forward-converter.md`**: Forward converter operating mechanics, tertiary demagnetizing winding reset, two-switch forward, output inductor filter, and duty-cycle constraints (D < 50\%).

**`isolated/flyback-converter.md`**: Flyback coupled inductor, Discontinuous (DCM) vs Continuous (CCM) conduction, RCD clamp snubber calculation, and multi-output isolated generation.

**`isolated/dual-active-bridge.md`**: Dual Active Bridge (DAB) bidirectional power conversion, single/dual/triple phase-shift modulation (SPS/DPS/TPS), leakage inductance energy transfer, and Zero-Voltage Switching (ZVS) boundaries.

**`isolated/full-bridge-converter.md`**: Phase-Shifted Full-Bridge (PSFB), diagonal vs phase-shift modulation, soft-switching ZVS mechanisms, secondary diode ringing, and active clamp snubbers.

**`isolated/half-bridge-converter.md`**: Isolated half-bridge DC-DC converter, capacitive voltage divider, automatic core volt-second balancing, and switch voltage clamping to V_{IN.

**`isolated/push-pull-converter.md`**: Push-pull topology, center-tapped transformer, ground-referenced primary low-side switches, core saturation / flux walking hazard and active balance control.

**`isolated/resonant-converters.md`**: [HIGH IMPORTANCE] Deep dive into Series Resonant (SRC), Parallel Resonant (PRC), and LLC Resonant Half-Bridge converters. Resonant tank impedance (Z_r), quality factor (Q), inductance ratio (k), normalized voltage gain curves, and complete ZVS/ZCS design procedures.

Non-Isolated DC-DC Topologies:

**`non-isolated/buck-converter.md`**: Step-Down Buck converter, CCM/DCM boundary condition, inductor & capacitor ripple formulations, synchronous rectification, and closed-loop control.

**`non-isolated/boost-converter.md`**: Step-Up Boost converter, Right-Half-Plane (RHP) zero stability challenge, inductor energy balance, and synchronous boost design.

**`non-isolated/buck-boost-converter.md`**: Inverting buck-boost and 4-switch synchronous non-inverting buck-boost topologies, operational mode transitions, and component sizing.

**`non-isolated/cuk-converter.md`**: Cuk converter, capacitive energy transfer, continuous non-pulsating input and output currents, low EMI signature, and coupled-inductor zero-ripple operation.

**`non-isolated/sepic-converter.md`**: Single-Ended Primary-Inductor Converter (SEPIC), non-inverting buck-boost operation, series DC blocking capacitor protecting against output short circuits.

**`non-isolated/zeta-converter.md`**: Zeta (Inverse SEPIC) converter, continuous output inductor current, capacitive energy transfer, and low output voltage ripple.