Isolated Push-Pull DC-DC Converter: Topology, Flux Walking & Low-Voltage Design

Welcome to the VVDN Engineering Hub Technical Dossier on the Push-Pull DC-DC Converter. This guide provides an exhaustive hardware engineering analysis of the Push-Pull topology, covering its center-tapped primary structure, ground-referenced dual low-side switching, the severe Transformer Flux Walking dynamic core saturation hazard, 2 \cdot V_{IN switch voltage stress, and component sizing rules for low-voltage battery-fed applications.

1. Operating Principle & Push-Pull Architecture

The Push-Pull Converter utilizes a center-tapped transformer primary winding powered by two ground-referenced switches operated alternately:

Push-Pull DC-DC Converter Power Stage +Vin DC ────────────────────────┬────────────────────────────────────────────────────────┐ │ │ Transformer Primary ┌┴┐ Center-Tap Connection │ │ C_in │ └┬┘ ┌───────────┴───────────┐ │ │ │ │ [ Np1 ] [ Np2 ] │ ││ ││ │ Drain (D) ────┤ ├──── Drain (D) │ ┌───┴───┐ │ │ ┌───┴───┐ │ │ Q1 │ │ │ │ Q2 │ │ └───┬───┘ │ │ └───┬───┘ │ │ │ │ │ │ │ Source │ │ │ Source │ GND_PRI ──┴─────────┴───────────────────────┴─────────┴──────────────────────────────────┴─── GND_PRI ││ ││ =================== │ ===================== │ =================== ISOLATION BARRIER =================== ││ ││ └───[ Ns1 ]───[>¦] D1 ──┬───[ Inductor Lo ]──┬───> +Vout ││ │ │ GND_SEC ──┼─────────────────┤ ┌┴┐ ││ │ │ │ Co [ Ns2 ]───[>¦] D2 ──┘ └┬┘ ││ │ GND_SEC ──┴──────────────────────────────────────┴─── GND_SEC

1.1 Conduction Intervals:

Interval 1: Switch Q1 ON ($0 < t \le D \cdot T_s$):

Switch Q_1 turns ON, pulling the bottom terminal of winding N_{p1 to ground (0\,V).

The full DC input V_{IN is impressed across N_{p1.

Secondary winding N_{s1 generates a positive voltage, forward-biasing diode D_1 and charging the output filter inductor L_o.

Autotransformer Action on Q2: The center-tapped primary behaves as an autotransformer. The voltage induced across N_{p2 is equal to +V_{IN. Therefore, the drain of the non-conducting switch Q_2 swings to:

V_{DS,Q2} = V_{IN} + V_{Np2} = 2 \cdot V_{IN}

Interval 2: Dead-Time ($D \cdot T_s < t \le 0.5 T_s$):

Both switches Q_1 and Q_2 are OFF. Primary currents drop to zero.

Secondary diodes D_1 and D_2 both conduct simultaneously, freewheeling the output inductor current I_{Lo and clamping the secondary to 0\,V.

Interval 3: Switch Q2 ON ($0.5 T_s < t \le (0.5 + D) \cdot T_s$):

Switch Q_2 turns ON, impressing V_{IN across winding N_{p2 in the opposite magnetic orientation.

Diode D_2 conducts, supplying output current through L_o.

The drain of switch Q_1 now sees 2 \cdot V_{IN.

Interval 4: Dead-Time ($(0.5 + D) \cdot T_s < t \le T_s$):

Both switches OFF; secondary diodes freewheel until cycle repeats.

2. Voltage and Current Waveforms

Push-Pull Key Operating Waveforms Gate Q1 ───┐ ┌──────────────┐ └────────┘ └───────────────────────────── Gate Q2 ───────────────────────────┐ ┌──────────────┐ ───────────────────────────┘ └──────────────┘ │◄─ D ─►│ V_DS,Q1 2*Vin ─────────────────────┐ ┌──────────────────── │ │ ┌────────┐ └────────┘ ───┘ └──────────────────────────────────────────── V_DS,Q2 ┌────────┐ ───┐ ┌──────────────┘ └──────────────────── │ │ └────────┘ I_Lo / \ / \ / \ ───/ \────────────────────/ \────────────────────/ \

3. The Fatal Hazard: Transformer Flux Walking

In a Push-Pull converter, there is no series DC blocking capacitor because each primary winding is tied directly to the positive DC rail.

Dynamic Flux Walking Towards Core Saturation B (Flux Density) ^ +B_sat ┼─────────────────────── Core Saturation Boundary │ .◄────── Progressive Drift (Cycle N) │ /│ │ / │ │ Cycle 2 / │ │ / │ │ Cycle 1 / │ │ / │ -H (Field Force) ─┼────────/──────┼───────────────> +H │ / │ │ / │ -B_sat ┼─────'─────────┴───────

3.1 The Physical Mechanism of Flux Walking:

If there is the slightest mismatch in:

Switch turn-on or turn-off propagation delays (\Delta t = t_{on,Q1 - t_{on,Q2)

Switch on-state resistances (R_{DS(on),Q1 \ne R_{DS(on),Q2)

Winding DC copper resistances (R_{cu,p1 \ne R_{cu,p2)

The volt-seconds applied across the transformer during positive and negative half-cycles will not balance:

\Delta (V \cdot t) = \int_0^{D T_s} V_{p1}(t)\,dt - \int_{0.5 T_s}^{(0.5+D) T_s} V_{p2}(t)\,dt \ne 0

This non-zero net DC volt-second integral acts like a DC voltage source applied directly to the magnetizing inductance. Every switching cycle, the operating flux density walks higher along the B-H curve until the core reaches +B_{sat. When saturated:

Magnetizing inductance collapses: L_m \rightarrow 0.

The primary switch current spikes exponentially to hundreds of amperes.

Catastrophic MOSFET thermal overstress occurs within milliseconds.

3.2 Hardware Solutions to Eliminate Flux Walking:

Peak Current-Mode Control (PCMC): Mandatory for Push-Pull converters. By terminating each switch's on-time when the peak primary switch current hits the control threshold, PCMC enforces identical peak magnetic flux excursions on alternate half-cycles, dynamically stabilizing the flux around zero.

Small Transformer Air Gap: Inserting a small physical air gap in the ferrite core lowers the effective magnetic permeability µ_r and tilts the B-H loop, dramatically increasing the saturation flux margin at the cost of higher magnetizing current.

Primary RC / RCD Snubbers: Absorb leakage-inductance voltage spikes that could trigger premature breakdown.

4. Mathematical Formulations & Component Sizing

4.1 Output Voltage Conversion Ratio:

V_{out} = 2 \cdot \frac{N_s}{N_p} \cdot V_{IN} \cdot D

Where D ≤ 0.45 per switch (total duty ratio 2D < 0.90 to avoid overlap cross-conduction).

4.2 Switch Voltage Rating:

Because of transformer autotransformer action and leakage inductance ringing:

V_{DS,pk} = 2 \cdot V_{IN,max} + V_{spike}

Where V_{spike = I_{pri,pk \cdot \sqrt{\frac{L_{lk{C_{oss.

Design Rule: Specify MOSFET drain rating with at least a 30\% safety derating:

V_{DS,rating} \ge 2.6 \cdot V_{IN,max}

4.3 Primary RMS Current:

I_{pri,rms} = \frac{N_s}{N_p} \cdot I_o \cdot \sqrt{D}

5. Engineering Verdict & Application Domain

| Parameter | Push-Pull Assessment |
| --- | --- |
| Driver Simplicity | Best in class: Dual ground-referenced low-side drivers (no high-side level shifters or bootstrap circuits). |
| Switch Voltage Stress | Poor: 2 \cdot V_{IN + spike. Unsuitable for universal AC/DC (400\,V rail would require 1000\,V+ switches). |
| Input Voltage Domain | Ideal for low-voltage DC rails: 12\,V, 24\,V, 48\,V battery inputs (automotive, solar off-grid, telecom inverter front-ends). |
| Transformer Utilization | High: Full two-quadrant excitation (± \Delta B). Primary winding requires bifilar center-tapped winding to minimize leakage mismatch. |
| Control Complexity | Requires cycle-by-cycle Peak Current Mode Control to prevent fatal flux walking. |