Isolated Resonant DC-DC Converters: SRC, PRC & LLC Half-Bridge Masterclass

Welcome to the VVDN Engineering Hub Technical Dossier on Isolated Resonant DC-DC Converters. Resonant power conversion is the gold standard for high-density, ultra-high-efficiency power systems—including 80 PLUS Titanium server power supplies, telecom rectifiers, electric vehicle (EV) DC-DC converters, and renewable energy interfaces.

This document explores the mathematical principles of resonant tanks, compares Series Resonant Converters (SRC) and Parallel Resonant Converters (PRC), and conducts a deep dive into the industry-dominant LLC Resonant Half-Bridge Converter, including First Harmonic Approximation (FHA), gain curves M(f_n, Q, L_n), ZVS/ZCS boundary conditions, and a complete hardware design methodology.

1. Why Resonant Conversion? Hard vs. Soft Switching

Conventional PWM converters (Forward, Flyback, Full-Bridge) suffer from hard switching: switches turn on and turn off while carrying full voltage and current simultaneously.

Hard Switching vs. Soft Switching (ZVS) HARD SWITCHING (Turn-On): SOFT SWITCHING / ZVS (Turn-On): V_DS I_D V_DS I_D ^ ^ ^ ^ │\ /│ │\ │ │ \ / │ Overlapping Area │ \ │ V_DS reaches 0V │ \/ │ = Turn-On Energy │ \ │ BEFORE I_D rises! │ /\ │ Loss (E_on) │ \ │ E_on = 0 Joules! │ / \ │ │ \_│______ 0───────┴─────────> Time 0──────┴─────────────> Time

Limitations of Hard-Switched PWM:

Switching Losses: P_{sw = ≤ft( E_{on + E_{off \right) \cdot f_s. As switching frequency increases above 100\,kHz, switching losses dominate, severely limiting power density.

Capacitive Turn-On Loss: Energy stored in MOSFET parasitic drain-source capacitance (\frac{1{2 C_{oss V_{DS^2) is dissipated internally as heat every turn-on cycle.

Diode Reverse Recovery: Secondary diodes snapped off at high di/dt suffer reverse-recovery current spikes, inducing ringing and electromagnetic interference (EMI).

The Resonant Solution:

Resonant converters introduce an LC tank network that shapes the switch voltage or current into smooth sinusoids:

Zero-Voltage Switching (ZVS): Voltage across the switch drops to zero before gate drive turn-on, eliminating E_{on and C_{oss losses.

Zero-Current Switching (ZCS): Current through the switch or rectifier diode drops to zero before turn-off, eliminating turn-off losses and reverse-recovery phenomena.

2. Resonant Converter Taxonomy: SRC vs. PRC vs. LLC

Resonant Converter Topologies Series Resonant (SRC) Parallel Resonant (PRC) LLC Resonant (LLC) Cr Lr Cr Lr Cr Lr +Vin ────[ ]───^^^^──┐ +Vin ────[ ]───^^^^──┬─── +Vin ────[ ]───^^^^──┬─── │ │ │ ( ) Xfmr ┌┴┐ ┌┴┐ ( ) Primary │ │ Cp │ │ Lm (Magnetizing) │ └┬┘ └┬┘ │ │ │ │ ( ) Xfmr ( ) Xfmr │ ( ) Primary ( ) Primary GND ─────────────────┴─── GND ─────────────────┴─── GND ─────────────────┴───

Comprehensive Comparison:

| Feature / Metric | Series Resonant Converter (SRC) | Parallel Resonant Converter (PRC) | LLC Resonant Converter |
| --- | --- | --- | --- |
| Resonant Tank Elements | L_r, C_r (2 elements) | L_r, C_p (2 elements) | L_r, C_r, L_m (3 elements) |
| No-Load / Light-Load Regulation | Fails: At no load (R_L \rightarrow \infty), gain is fixed at 1; frequency must go to \infty. | Good: Can regulate down to zero load with finite frequency change. | Excellent: Regulates seamlessly from full load down to zero load. |
| Circulating Energy at Light Load | Minimal (current drops with load). | Extremely high (tank current flows through C_p regardless of load). | Minimal (magnetizing current circulates only enough to guarantee ZVS). |
| Output Filter | Capacitive (C_o only). | Inductive (L_o + C_o). | Capacitive (C_o only, no inductor). |
| Efficiency at Wide Range | Poor across wide V_{IN variations. | Poor at light load. | Peak Industry Standard (> 97\%). |

3. The LLC Resonant Half-Bridge Converter: Architecture & Physics

The LLC Resonant Converter utilizes three reactive elements: a series resonant capacitor C_r, a series resonant inductor L_r (often integrated as transformer primary leakage inductance), and the transformer primary magnetizing inductance L_m:

LLC Resonant Half-Bridge Converter Power Stage +Vin DC ────┬─────────────────────────────────────────────────────────────────────────────────┐ │ │ Drain (D) ┌┴┐ ┌───┴───┐ Q1 │ │ C_in │ S1 │ High-Side Switch └┬┘ └───┬───┘ │ │ Source (S) │ ├─── Midpoint (HB) ──┐ │ │ │ │ Drain (D) ┌─┴─┐ Cr (Resonant Cap) │ ┌───┴───┐ Q2 │ │ │ │ S2 │ Low-Side └─┬─┘ │ └───┬───┘ Switch ├───[ Lr (Resonant Inductor) ]───┬──────────────────┐ │ │ Source (S) │ │ │ │ GND_PRI ────┴────────────────────┼───────────────────────────────┌┴┐ Lm │ │ │ │ │ (Magnetizing) │ │ │ └┬┘ │ │ │ │ │ │ │ [ Primary Winding Np ] │ │ │ ││ │ │ └────────────────────────────────┼──────────────────┘ │ ││ │ ================================================ ISOLATION BARRIER ================================================ ││ Secondary ││ Center-Tap Winding ┌───[ Ns1 ]───[>¦] D1 ──┬───> +Vout │ ││ │ GND_SEC ├───┼───────────────────┤ │ ││ │ └───[ Ns2 ]───[>¦] D2 ──┤ ││ │ ┌┴┐ Co (Only │ │ Cap Needed!) └┬┘ GND_SEC ────────────────────────┴─── GND_SEC

3.1 Two Characteristic Resonant Frequencies:

Series Resonant Frequency ($f_r$ or $f_0$):

Determined by the resonant tank L_r and C_r:

f_r = \frac{1}{2\pi \sqrt{L_r \cdot C_r}}

Lower Resonant Frequency ($f_m$ or $f_p$):

Determined when L_m is liberated from secondary clamping (magnetizing current resonates with C_r):

f_m = \frac{1}{2\pi \sqrt{(L_r + L_m) \cdot C_r}} = \frac{f_r}{\sqrt{1 + L_n}}

Where L_n = \frac{L_m{L_r is the inductance ratio.

4. First Harmonic Approximation (FHA) & Voltage Gain Formulation

Under First Harmonic Approximation (FHA), the square-wave voltages and currents are modeled by their fundamental sinusoidal Fourier components:

LLC Resonant Equivalent AC Circuit (FHA Model) Cr Lr ───o─────[ ]────────^^^^^^───────┬────────────────o─── + │ + ┌┴┐ v_ac,in │ │ Lm v_ac,out (R_ac) └┬┘ - │ - ───o───────────────────────────────┴────────────────o───

4.1 Equivalent AC Resistance (R_{ac):

Reflecting the DC load resistance R_L = \frac{V_o{I_o across the secondary rectifier and transformer turns ratio n = \frac{N_p{N_s:

R_{ac} = \frac{8 \cdot n^2}{\pi^2} \cdot R_L

4.2 Normalized Parameters:

Normalized switching frequency: f_n = \frac{f_s{f_r

Inductance ratio: L_n = \frac{L_m{L_r (typically between 3 \dots 8)

Quality factor: Q = \frac{\sqrt{L_r / C_r{R_{ac = \frac{Z_o{R_{ac

4.3 DC Voltage Gain Formula:

The transfer function of the LLC resonant tank is:

M(f_n, Q, L_n) = \left| \frac{v_{ac,out}}{v_{ac,in}} \right| = \frac{1}{\sqrt{\left[ 1 + \frac{1}{L_n} \left( 1 - \frac{1}{f_n^2} \right) \right]^2 + Q^2 \left( f_n - \frac{1}{f_n} \right)^2}}

5. LLC Operating Regions & Gain Curve Analysis

LLC Resonant Converter Gain Curves Gain M ^ │ Q = 0.2 (Light Load) 1.6 ┼ .-'""'-. │ .' '. Q = 0.5 (Nominal) 1.4 ┼ / /\ \ │ / / \ \ Q = 1.0 (Full Load) 1.2 ┼ / / \ '. │ / / \ \ Unity 1.0┼──────────/────/────────\───────\──────── Resonance (fn = 1.0) │ / / \ \ 0.8 ┼ / / \ \ │ / / \ \ 0.6 ┼──────'────'────────────────'───────'──── │ │ │ 0 f_m f_r (fn=1.0) ───> Normalized Frequency (fn) │◄── Capacitive ──►│◄─── Inductive ZVS Zone ────►│ (FORBIDDEN ZONE)

The Three Operating Modes:

Resonance Operation ($f_s = f_r \implies f_n = 1.0$):

The tank impedance is purely resistive (Z_r = 0).

Gain is identically unity ($M = 1.0$) independent of load Q.

Primary switches achieve perfect ZVS turn-on.

Secondary rectifier diodes turn off at zero current (ZCS), completely eliminating diode reverse recovery losses!

This is the highest efficiency operating point (typically designed for nominal input voltage).

Below Resonance ($f_m < f_s < f_r \implies f_n < 1.0$) [Boost Mode]:

Gain M > 1.0. The converter boosts output voltage to compensate for low V_{IN (e.g., during line sag or battery discharge).

Tank current resonates and falls to the magnetizing current I_m before the half-cycle ends.

Secondary diode current terminates naturally to zero during the cycle: perfect secondary ZCS.

Primary MOSFETs maintain ZVS turn-on.

Above Resonance ($f_s > f_r \implies f_n > 1.0$) [Buck Mode]:

Gain M < 1.0. The converter steps down output voltage for high V_{IN.

Primary MOSFETs maintain ZVS turn-on.

Secondary diode current is continuous and snapped off at switch turn-off: ZCS is lost on secondary (reverse-recovery occurs, requiring ultra-fast diodes or Schottky diodes).

Capacitive Region ($f_s < f_m$) — THE FORBIDDEN ZONE:

Tank current leads switch voltage. Primary switches lose ZVS and suffer hard turn-on.

Body diodes of the MOSFETs conduct reverse recovery current directly into the incoming switch, causing catastrophic switch shoot-through failure.

Modern LLC controller ICs (e.g., UCC25640x, HR1001) integrate Capacitive Mode Prevention (CMP) logic to clamp minimum switching frequency above the capacitive threshold.

6. Step-by-Step LLC Resonant Hardware Design Procedure

Step 1: Establish Converter Specifications

Input Voltage: V_{IN,nom = 390\,V, V_{IN,min = 320\,V, V_{IN,max = 420\,V (PFC Rail)

Output: V_o = 12\,V, I_o = 25\,A (P_o = 300\,W)

Resonant frequency target: f_r = 100\,kHz

Step 2: Transformer Turns Ratio (n)

Design for unity gain M_{nom = 1.0 at nominal input V_{IN,nom:

n = \frac{N_p}{N_s} = \frac{V_{IN,nom}}{2 \cdot (V_o + V_F)} = \frac{390\,\text{V}}{2 \cdot (12\,\text{V} + 0.4\,\text{V})} \approx 15.7 \implies \text{Select } n = 16

Step 3: Calculate Required Minimum and Maximum Gain

M_{max} = \frac{n \cdot (V_o + V_F)}{V_{IN,min} / 2} = \frac{16 \cdot 12.4\,\text{V}}{320\,\text{V} / 2} = \frac{198.4}{160} = 1.24

M_{min} = \frac{n \cdot (V_o + V_F)}{V_{IN,max} / 2} = \frac{16 \cdot 12.4\,\text{V}}{420\,\text{V} / 2} = \frac{198.4}{210} = 0.94

Add 10\% design margin: M_{peak,req = 1.1 \cdot M_{max = 1.36.

Step 4: Select Inductance Ratio (L_n) and Quality Factor (Q)

An inductance ratio L_n = \frac{L_m{L_r = 5 provides an excellent balance between wide voltage regulation and low circulating current.

From FHA gain curves for L_n = 5 and M_{peak = 1.36, choose maximum full-load quality factor:

Q_{max} = 0.40

Step 5: Calculate Equivalent Load and Resonant Tank Values

R_L = \frac{V_o}{I_o} = \frac{12\,\text{V}}{25\,\text{A}} = 0.48\,\Omega

R_{ac} = \frac{8 \cdot n^2}{\pi^2} \cdot R_L = \frac{8 \cdot 16^2}{\pi^2} \cdot 0.48 = \frac{2048 \cdot 0.48}{9.8696} \approx 99.6\,\Omega

Resonant Capacitor ($C_r$):

C_r = \frac{1}{2\pi \cdot f_r \cdot Q \cdot R_{ac}} = \frac{1}{2\pi \cdot 100\,000 \cdot 0.40 \cdot 99.6} \approx 40\,\text{nF} \implies \text{Select } 39\,\text{nF (Polypropylene Film)}

Resonant Inductor ($L_r$):

L_r = \frac{1}{(2\pi \cdot f_r)^2 \cdot C_r} = \frac{1}{(2\pi \cdot 100\,000)^2 \cdot 39 \cdot 10^{-9}} \approx 65\,\mu\text{H}

Magnetizing Inductance ($L_m$):

L_m = L_n \cdot L_r = 5 \cdot 65\,\mu\text{H} = 325\,\mu\text{H}

Step 6: Verify Primary ZVS Condition During Dead-Time

To guarantee ZVS at switch turn-on, the magnetizing current peak I_{m,pk must discharge the two MOSFET output capacitances (2 C_{oss) during dead-time t_d:

I_{m,pk} = \frac{n \cdot V_o}{4 \cdot L_m \cdot f_r} = \frac{16 \cdot 12\,\text{V}}{4 \cdot 325\,\mu\text{H} \cdot 100\,000\,\text{Hz}} = \frac{192}{130} \approx 1.48\,\text{A}

Required dead-time t_{d,min:

t_d \ge \frac{2 \cdot C_{oss} \cdot V_{IN}}{I_{m,pk}} = \frac{2 \cdot 150\,\text{pF} \cdot 400\,\text{V}}{1.48\,\text{A}} \approx 81\,\text{ns}

Setting dead-time t_d = 250\,ns \dots 350\,ns guarantees complete, robust ZVS switching across all line and load variations.

7. Practical Engineering Summary

Transformer Integration: Rather than using a separate physical inductor for L_r, wind the transformer with intentional spatial separation or a magnetic shunt between primary and secondary sections to integrate L_r directly as transformer leakage inductance (L_{lk = L_r).

Resonant Capacitor Selection: The resonant capacitor carries full resonant AC current (I_{Cr,rms \approx 2 \dots 4\,A). Never use ceramic MLCCs (due to DC bias capacitance drop and acoustic microphonics) or polyester film. Always use High-Current Metalized Polypropylene (MKP) film capacitors.

Synchronous Rectification (SR): On the low-voltage high-current secondary (12\,V, 25\,A), replace Schottky diodes with low R_{DS(on) MOSFETs (< 2\,m\Omega) driven by specialized resonant SR controllers (e.g., TEA1995, MP6924) sensing drain-source V_{DS ringing.