Step-Up (Boost) DC-DC Converter: Right-Half-Plane Zero, CCM/DCM & Design

Welcome to the VVDN Engineering Hub Technical Dossier on the Step-Up (Boost) DC-DC Converter. The Boost converter is fundamental to battery-powered electronics, solar maximum power point tracking (MPPT), LED backlight drivers, and Active Power Factor Correction (PFC) pre-regulators.

This document explores the magnetic physics of boost energy transfer, derives the infamous Right-Half-Plane (RHP) Zero stability limitation, details diode reverse-recovery phenomena, and provides practical component selection criteria.

1. Operating Principle & Boost Topology

The Boost Converter steps up an input DC voltage V_{IN to a higher DC output voltage V_o by utilizing an inductor to store energy from the source and discharge it in series with the source into the load:

Step-Up (Boost) DC-DC Converter Inductor L +Vin DC ─────────────^^^^^^──────────────┬───────────────[>¦] D1 (Boost Diode) ──┬───> +Vout │ │ Drain (D) ┌┴┐ ┌───┴───┐ Q1 (Low-Side Switch) │ │ C_out │ SW │ └┬┘ └───┬───┘ │ │ Source (S) │ GND ─────────────────────────────────────┴───────────────────────────────────────┴─── GND

1.1 Switching Intervals (CCM):

Interval 1: Switch ON ($0 < t \le D \cdot T_s$):

MOSFET Q_1 conducts, connecting the switching node SW to GND (0\,V).

Diode D_1 is reverse-biased (V_{diode = -V_o).

Full input voltage V_{IN appears across inductor L. Current ramps up linearly:

\frac{di_L}{dt} = \frac{V_{IN}}{L}

Energy is drawn from the input source and stored in L.

Crucial Dynamic Observation: During this interval, the output load is completely disconnected from the source and inductor; output current I_o is supplied solely by the discharging output capacitor C_{out.

Interval 2: Switch OFF ($D \cdot T_s < t \le T_s$):

Q_1 turns OFF. The collapsing magnetic field forces the voltage across L to reverse polarity.

Node SW flies above V_o, forward-biasing diode D_1.

Voltage across L is V_L = V_{IN - V_o < 0. Current decays linearly:

\frac{di_L}{dt} = \frac{V_{IN} - V_o}{L}

Energy stored in L, along with additional energy from input V_{IN, flows into C_{out and the load.

2. Voltage and Current Waveforms

Continuous Conduction Mode (CCM) Waveforms V_SW ^ Vo ┼──────────────┐ ┌────────────── │ │ │ 0V ┼──────────────┴──────────────────────┴──────────────> Time │◄─── D*Ts ───►│◄──── (1-D)*Ts ──────►│ i_L ^ │ / \ / \ I_max ┼─────────────/ \──────────────────/ \─────────── I_avg ┼─ ─ ─ ─ ─ ─ /─ ─ ─\─ ─ ─ ─ ─ ─ ─ ─ /─ ─ ─\─ ─ ─ ─ ─ (I_in = Io / (1-D)) I_min ┼───────────/ \──────────────/ \───────── 0────────────────────────────────────────────────────> Time i_Diode ^ I_max ┼──────────────┐ ┌────────────── │ │\ │\ 0A ┼──────────────┴─\────────────────────┴─\────────────> Time │◄── Zero ────►│◄─ Pulsating Current ─►│

3. Mathematical Formulations & Component Sizing

3.1 Voltage Conversion Ratio:

Applying volt-second balance across inductor L:

\int_0^{T_s} v_L(t) \, dt = (V_{IN}) \cdot D \cdot T_s + (V_{IN} - V_o) \cdot (1 - D) \cdot T_s = 0

V_{IN} \cdot D + V_{IN} - V_o - V_{IN} \cdot D + V_o \cdot D = 0

V_{IN} = V_o (1 - D) \implies V_o = \frac{V_{IN}}{1 - D}

D = 1 - \frac{V_{IN}}{V_o}

3.2 Inductor Ripple & Value Calculation:

\Delta I_L = \frac{V_{IN} \cdot D}{f_s \cdot L}

Average inductor current equals average input current:

I_{L,avg} = I_{IN} = \frac{I_o}{1 - D} = \frac{V_o \cdot I_o}{\eta \cdot V_{IN}}

Choosing ripple ratio r = \frac{\Delta I_L{I_{L,avg \approx 0.2 \dots 0.4:

L = \frac{V_{IN} \cdot (V_o - V_{IN})}{f_s \cdot \Delta I_L \cdot V_o} = \frac{V_{IN} \cdot D}{f_s \cdot (r \cdot I_{IN})}

3.3 Output Capacitor Sizing & RMS Ripple Stress:

Because diode current is discontinuous, C_{out must support the entire load current I_o during switch ON-time:

\Delta V_{o,cap} = \frac{I_o \cdot D}{f_s \cdot C_{out}} \implies C_{out} \ge \frac{I_o \cdot D}{f_s \cdot \Delta V_{o,allowable}}

The RMS ripple current through C_{out is exceptionally severe:

I_{Cout,rms} = I_o \cdot \sqrt{\frac{D}{1 - D}}

Engineering Implication: Always verify that the chosen capacitor's RMS ripple current rating exceeds I_{Cout,rms to prevent thermal degradation.

4. The Right-Half-Plane (RHP) Zero Hazard & Stability

The Boost converter possesses an intrinsic non-minimum phase zero in its small-signal control-to-output transfer function, known as the Right-Half-Plane (RHP) Zero:

\omega_{RHPZ} = \frac{R_{load} \cdot (1 - D)^2}{L} \quad (\text{rad/s}) \implies f_{RHPZ} = \frac{R_{load} \cdot (1 - D)^2}{2\pi \cdot L} \quad (\text{Hz})

Transient Response to a Step Increase in Duty Cycle Duty Cycle D ^ │ Step increase in D (Load asks for more Vo) ├───────┌───────────────────────────────────────────────> Time Output Vo ^ │ Vo initially DIPS! │ - - ┐ .------------------------ │ \ / │ '--------------' (Takes time for L to charge) 0───────────────────────────────────────────────────────> Time

4.1 Physical Explanation of the RHP Zero:

When load increases, the controller increases duty cycle D to boost output voltage.

An increased D means the switch stays ON longer, which reduces the diode conduction time ($1-D$).

Consequently, less energy is delivered to the output during the immediate next switching cycles while inductor current is ramping up!

The output voltage drops initially before rising to its target.

In the frequency domain, an RHP zero provides $+20\,\text{dB/decade}$ gain boost but $-90^\circ$ phase lag—a toxic combination for closed-loop stability.

4.2 Control Bandwidth Restriction:

To ensure stable feedback and avoid 180^\circ phase margin collapse, the crossover frequency (f_c) of the voltage control loop must be capped at:

f_c \le \frac{1}{3} \dots \frac{1}{5} \cdot f_{RHPZ}

Worst-case operating condition: Minimum input voltage V_{IN,min and maximum load current I_{o,max (minimum R_{load and highest D).

5. Diode Reverse Recovery & Synchronous Boost

In CCM, when switch Q_1 turns ON, diode D_1 is conducting full inductor current.

Standard silicon PN diodes take t_{rr = 30 \dots 100\,ns to clear reverse minority carriers.

During t_{rr, the diode conducts reverse current directly from V_o through Q_1 to GND, creating a massive current spike (I_{spike = I_L + I_{rr,pk) and severe turn-on losses in Q_1.

Hardware Solutions:

Use Silicon Carbide (SiC) Schottky Diodes (Q_{rr \approx 0).

Implement Synchronous Boost: Replace diode with an active N-channel or P-channel MOSFET with body-diode anti-parallel conduction and dead-time management.