Buck-Boost DC-DC Converters: Inverting vs 4-Switch Non-Inverting Architecture

Welcome to the VVDN Engineering Hub Technical Dossier on Buck-Boost DC-DC Converters. When the input DC voltage can be higher, equal to, or lower than the desired regulated output rail—such as automotive battery cranking (6\,V \dots 28\,V to 12\,V), lithium-ion battery discharge profiles (2.7\,V \dots 4.2\,V to 3.3\,V), or USB Power Delivery (5\,V \dots 20\,V)—the Buck-Boost topology is mandatory.

This guide provides an exhaustive hardware analysis of the classic Inverting Buck-Boost Converter and the modern industrial workhorse: the 4-Switch Synchronous Non-Inverting Buck-Boost Converter (FSBB).

1. Classic Inverting Buck-Boost Converter

The classic single-switch Buck-Boost converter produces an output voltage whose polarity is inverted with respect to the input source ground:

Classic Inverting Buck-Boost Converter +Vin DC ────┬────────────────── Drain (D) │ ┌───┴───┐ Q1 (High-Side Switch) ┌┴┐ │ SW │ │ │ C_in └───┬───┘ └┬┘ │ Source (S) │ ├─── Switching Node (SW) ───[¦<] D1 (Diode) ──┬───> -Vout │ │ │ │ ┌┴┐ ┌┴┐ │ │ │ Inductor L │ │ C_out │ └┬┘ └┬┘ │ │ │ GND ────────┴───────────────────────┴─────────────────────────────────────────────┴─── GND

1.1 Operating Mechanism:

Switch ON ($0 < t \le D \cdot T_s$):

Q_1 is ON; Diode D_1 is reverse-biased by -V_o and V_{IN.

Inductor L is connected directly between V_{IN and GND. Inductor current ramps up:

\frac{di_L}{dt} = \frac{V_{IN}}{L}

Energy is stored in inductor L. Capacitor C_{out independently supplies the load.

Switch OFF ($D \cdot T_s < t \le T_s$):

Q_1 turns OFF. The inductor forces node SW negative to maintain continuous current flow.

Diode D_1 is forward-biased, pulling current out of the negative output rail through the inductor to GND.

Inductor current decays:

\frac{di_L}{dt} = \frac{-|V_o|}{L}

Output voltage is strictly negative with respect to GND.

1.2 Transfer Function & Volt-Second Balance:

\int_0^{T_s} v_L(t) \, dt = V_{IN} \cdot D \cdot T_s + (-|V_o|) \cdot (1 - D) \cdot T_s = 0

|V_o| = V_{IN} \cdot \frac{D}{1 - D} \implies V_o = -V_{IN} \cdot \frac{D}{1 - D}

When D < 0.5 \implies |V_o| < V_{IN (Step-Down / Buck Mode)

When D = 0.5 \implies |V_o| = V_{IN (Unity Gain)

When D > 0.5 \implies |V_o| > V_{IN (Step-Up / Boost Mode)

1.3 Severe Switch Stress:

Both the switch Q_1 and the diode D_1 see a peak reverse voltage equal to the sum of input and output voltages:

V_{stress} = V_{IN} + |V_o|

Engineering Drawback: The negative output polarity and severe switch voltage stress make the classic topology inconvenient for standard single-rail consumer and automotive electronics.

2. Four-Switch Synchronous Non-Inverting Buck-Boost (FSBB)

The 4-Switch Buck-Boost (FSBB) combines a synchronous buck leg and a synchronous boost leg connected via a single central power inductor:

4-Switch Synchronous Non-Inverting Buck-Boost (FSBB) +Vin ───┬───────────────┐ ┌───────────────┬───> +Vout │ │ │ │ Drain (D) Drain (D) Drain (D) Drain (D) ┌───┴───┐ QA ┌───┴───┐ QB ┌───┴───┐ QC ┌───┴───┐ QD │ HS1 │ │ LS1 │ │ LS2 │ │ HS2 │ └───┬───┘ └───┬───┘ └───┬───┘ └───┬───┘ │ Source │ Source │ Source │ Source ├── Node SW1 ───┴─────[ Central Inductor L ]────┴── Node SW2 ───┤ ┌┴┐ ┌┴┐ Cin │ │ Cout│ │ └┬┘ └┬┘ GND ────┴───────────────────────────────────────────────────────────────┴─── GND

2.1 The Three Control Regimes:

To maximize efficiency (> 98\%), modern FSBB controllers (e.g., LM5176, LT8390, TPS55288) do not switch all 4 FETs simultaneously. Instead, they dynamically switch between three operating regions:

FSBB Operating Modes Across Input Voltage Sweep Efficiency 100% ┼──────────. .────────── │ '. .' 95% ┼ '-----------' │ (4-Switch Region) 0─────────────────────────────────────────> Vin Vin < Vo Vin ≈ Vo Vin > Vo [Pure Boost] [Buck-Boost] [Pure Buck]

Pure Buck Mode ($V_{IN} > 1.15 \cdot V_o$):

Q_D is held continuously ON (100\% duty cycle).

Q_C is held continuously OFF.

Q_A and Q_B switch as a standard synchronous buck converter.

Only 2 switches toggle; switching losses are minimal.

Pure Boost Mode ($V_{IN} < 0.85 \cdot V_o$):

Q_A is held continuously ON (100\% duty cycle).

Q_B is held continuously OFF.

Q_C and Q_D switch as a standard synchronous boost converter.

Buck-Boost Mode ($0.85 \cdot V_o \le V_{IN} \le 1.15 \cdot V_o$):

Both legs switch to provide seamless, monotonic output voltage regulation across the transition boundary without control loop instability or output voltage glitching.

3. Mathematical Formulations & Component Sizing

3.1 Inductor Sizing Formula:

The worst-case inductor ripple occurs in Boost mode at minimum input voltage:

L \ge \frac{V_{IN,min}^2 \cdot (V_o - V_{IN,min})}{f_s \cdot \Delta I_L \cdot V_o^2}

Choosing ripple current ratio r = 0.3 \dots 0.4 relative to maximum average inductor current:

I_{L,max} = \frac{V_o \cdot I_{o,max}}{\eta \cdot V_{IN,min}}

L = \frac{V_{IN,min} \cdot (V_o - V_{IN,min})}{f_s \cdot (r \cdot I_{L,max}) \cdot V_o}

3.2 Output Capacitor Sizing:

In boost and buck-boost modes, output current is discontinuous. C_{out must support the load current during switch ON time:

C_{out} \ge \frac{I_{o,max} \cdot D_{boost}}{f_s \cdot \Delta V_{o,allowable}}

4. Hardware Engineering Trade-Offs

| Parameter | Inverting Buck-Boost | 4-Switch Synchronous Buck-Boost (FSBB) |
| --- | --- | --- |
| Output Polarity | Negative (Inverted) | Positive (Common Ground) |
| Component Count | 1 Switch, 1 Diode, 1 Inductor | 4 MOSFETs, 1 Inductor, Dual Drivers |
| Switch Voltage Stress | $V_{IN} + | V_o |
| Peak Efficiency | 82\% \dots 88\% | 96\% \dots 98.5\% |
| Primary Applications | Low-cost negative bias rails (Op-Amps, LCDs) | USB-PD (5-20V), Automotive Infotainment, Drone ESCs |